CONVERSION TABLE
Listed below are conversion factors compiled by the Solid and Hazardous Waste Management Program to assist Recycling Coordinators in the calculation of tonnages when the materials are reported to them in volumes as opposed to weights.  
Paper



 UNCOMPACTED

 COMPACTED
01-Corrugated


40 lbs/cy



500 lbs/cy

50 cy = 1 ton


4 cy = 1 ton

02-Mixed Office Paper 

400 lbs/cy


755 lbs/cy 

5 cy = 1 ton


2.65 cy = 1 ton

03-Newspaper


500 lbs/cy


865 lbs/cy

4 cy = 1 ton


2.31 cy = 1 ton

Containers

05-Glass (loose) 


600 lbs/cy


1,500 lbs/cy

3.33 cy = 1 ton


1.33 cy = 1 ton

05-Glass (manually broken)
1,000 lbs/cy


1,500 lbs/cy

2 cy = 1 ton


1.33 cy = 1 ton

06-Aluminum Cans

74 lbs/cy



250 lbs/cy

27.02 cy = 1 ton


8 cy = 1 ton

07-Steel Cans


150 lbs/cy


350 lbs/cy

13.33 cy = 1 ton


5.71 cy = 1 ton

08-Plastic Cont 
(PETE)

30 lbs/cy



515 lbs/cy

66.66 cy = 1 ton


3.88 cy = 1 ton

08-Plastic Cont 
(HDPE)

25 lbs/cy



270 lbs/cy



80 cy = 1 ton


7.41 cy = 1 ton

Autos



WEIGHT/UNIT


UNITS/TON

12-Anti-freeze


7 lbs/gallon


285.71 gallons/ton

13-Batteries, Lead-Acid

33 lbs/battery


60.61 batteries/ton

15-Tires-Passenger

20 lbs/tire


100 tires/ton

15-Tires-Truck


90 lbs/tire


22.22 tires/ton

16-Used Motor Oil

7 lbs/gallon


285.71 gallons/ton

Yard Material



WEIGHT/UNIT


UNITS/TON
17-Brush-Chipped


500 lbs/cy


4 cy = 1 ton

17-Brush-Unchipped


250 lbs/cy


8 cy = 1 ton

18-Grass



1,111 lbs/cy


1.8 cy = 1 ton


Compacted

18-Grass



741 lbs/cy


2.7 cy = 1 ton


Uncompacted

19-Leaves



1,000 lbs/cy


2 cy = 1 ton

Compacted

19-Leaves



700 lbs/cy


2.86 cy = 1 ton


Vacuumed

19-Leaves



400 lbs/cy


5 cy = 1 ton


Loose

20-Stumps



500 lbs/cy


4 cy = 1 ton

Consumer Electronics

21-Central Processing Unit (CPU)

26 lbs = 1 CPU


77 units = 1 ton

21-Monitor



30 lbs = 1 Monitor


66.67 = 1 ton
21-Printer



25.33 lbs = 1 Printer

78.96 = 1 ton
21-Ink Cartridges



2.5 lbs = 1 Ink Cartridge

800 = 1 ton

Other
22-Concrete



4,000 lbs/cy


1 cy = 2 tons

22-Asphalt



115 lbs/sy 1"thick


17.39 sy = 1 ton


4140 lbs/cy


1 cy = 2.07 tons

23-Food Waste



412.5 lbs


4.85 drums = 1 ton

per 55 gal drum

24-Flourescent Lights


.62 lbs = 1 tube


3,200 tubes = 1 ton

29-Carpet



4.5 lbs/SqYrd


444.44 SqYrd = 1 Ton

30-Wood Scraps



285.71 lbs/cy


7 cy = 1 ton


Pallets

30-Wood Scraps



363.64 lbs/cy


5.5 cy = 1 ton

All Other

REMEMBER: 2,000 POUNDS EQUALS 1 TON.

WHATEVER YOU HAVE REPORTED IN POUNDS SIMPLY 

DIVIDE BY 2,000 TO GET YOUR TONS.
