# MODEL FIRE DEPARTMENT

RESPIRATORY PROTECTION

PROGRAM

## Revised February, 1999

New Jersey Department of Health and Senior Services

Occupational Health Service

Public Employees Occupational Safety and Health Program

The revised Public Employees Occupational Safety and Health (PEOSH) Respiratory Protection

Standard (29CFR1910.134) was adopted on September 21, 1998, with a compliance date of

March 21, 1999 (see Appendix G for a copy of the Respiratory Protection Standard). At the

same time, the PEOSH Standards for Firefighters (N.J.A.C. 12:100-10.10) were amended.

The amendments included:

- deleting the requirement that fire departments establish and maintain a respiratory

protection program meeting the requirements of ANSI Z88.5-1981, Practice for

Respiratory Protection for the Fire Service; and

- adding the requirement that fire departments establish and maintain a respiratory

protection program meeting the requirements of the revised PEOSH Respiratory

Protection Standard.

To assist fire departments in complying with PEOSH Respiratory Protection Standard

requirements, the New Jersey Department of Health and Senior Services, PEOSH Program and

the New Jersey Department of Labor, Office of Public Employees Safety have updated the

Model Fire Department Respiratory Protection Program. The Model Fire Department

Respiratory Protection Program must be tailored to the specific needs of each fire department.

The program alone does not ensure compliance with the respiratory protection requirements.

The program must be fully implemented as presented in the completed document.

This model program and other PEOSH information are available at the PEOSH website,

www.state.nj.us/health/eoh/peoshweb.

The PEOSH Program would like to receive your comments and suggestions about the Model

Fire Department Respiratory Protection Program. Our address is;

PEOSH Program

New Jersey Department of Health and Senior Services

PO Box 360

Trenton, New Jersey 08625-0360

Note:

The information contained in this document is not considered a substitute for any

provisions of the PEOSH Act or for any PEOSH standards.

Table of Contents

Section 1 - Introduction ........................                         ........................ 1

Section 2 - Standard Operating Procedures ....................         ............ 1

Section 3 - Training .............................................                            ...... 3

Section 4 - Respirator Fitting and Seal Check .....................       .......... 3

Section 5 - Inspection, Storage, Maintenance and Air Supply .............. 4

Section 6 - Medical Evaluation ..................................                   ......... 5

Section 7 - Recordkeeping ............................                      .................. 5

Section 8 - Program Evaluation .............................                  ............. 6

Appendix A - Manufacturers Instructions. ........................           .......... 7

Appendix B - Fit Test Protocol ..............................                   ............. 8

Appendix C - SCBA Training Outline ...........................              ........... 9

Appendix D - Fill Station Training Outline ....................          ............. 10

Appendix E - Inspection Forms ....................................                  ..... 11

Appendix F - Medical Evaluation Protocol .........................         ........ 13

Appendix G- PEOSH Respiratory Protection Standard ...................... 15

Prepared October 1995

1st Revision September 1997

2nd Revision February 1999

SECTION 1 - INTRODUCTION

Policy

It is the policy of the _________________ Fire Department to maintain comprehensive

occupational safety and health programs based upon sound engineering, education, and

enforcement. This document establishes Departmental policy, responsibilities, and

requirements for the protection of firefighters whose job requires the use of respiratory

protection.

This document will also provide assistance to the firefighter in the use and care of respiratory

protection.

The ________________Fire Department Safety Officer is__________________________.

He/she is solely responsible for all facets of this program and has full authority to make

necessary decisions to the ensure success of this program. The Safety Officer will develop

written detailed instructions covering each of the basic elements in this program, and is the sole

person authorized to amend these instructions.

SECTION 2 - STANDARD OPERATING PROCEDURES

General

Firefighters shall wear a self contained breathing apparatus (SCBA) under the following

conditions:

- while engaged in interior structural firefighting;

- while working in confined spaces where toxic products or an oxygen deficient

atmosphere may be present;

- during emergency situations involving toxic substances; and

- during all phases of firefighting and overhaul.

Firefighters wearing an SCBA must activate the personal alert safety system (PASS) device

before entering an area where respiratory protection is required.

Firefighters wearing SCBA shall conduct a seal check prior to each use.

Firefighters shall not remove the SCBA at any time in the dangerous atmosphere. SCBA shall

be used in accordance with the manufacturer’s instructions (see Appendix A).

All firefighters shall continue to wear an SCBA until the officer in charge determines that

respiratory protection is no longer required.

The use of the airline respirator located on _____________________ shall only be used for

exterior firefighting operations. The airline respirator shall be worn in combination with an

auxiliary SCBA or an appropriate escape type SCBA.

Protective Clothing

Firefighters wearing an SCBA shall be fully protected with the use of approved structural

firefighting clothing that meet the requirements of the PEOSH Standards for Firefighters

(N.J.A.C.12:100-10). Protective clothing shall include turnout coat, bunker pants, gloves, boots,

helmet, fire resistant hood, and PASS device.

Procedures for Interior Structural Firefighting

In interior structural fires, the fire department shall ensure that:

- At least two firefighters enter the immediately dangerous to life and health (IDLH)

atmosphere and remain in visual or voice contact with one another at all times;

- At least two firefighters will be located outside the IDLH atmosphere; and

- All firefighters engaged in interior structural firefighting will use SCBAs.

Note:

One of the two firefighters located outside the IDLH atmosphere may be assigned to an

additional role, such as incident commander in charge of the emergency or safety officer, so

long as the firefighter is able to perform assistance or rescue activities without jeopardizing the

safety or health of any firefighter working at the incident.

Nothing in this section is meant to preclude firefighters from performing emergency rescue

activities before an entire team has assembled.

There must always be at least two firefighters stationed outside during interior structural

firefighting. They must be trained, equipped, and prepared to enter if necessary to rescue

firefighters inside. However, the incident commander has the responsibility and flexibility to

determine when more than two outside firefighters are necessary given the circumstances of the

fire. The two-in/two-out rule does not require an arithmetic progression for every firefighter

inside, i.e. the rule should not be interpreted as four-in/four-out, eight-in/eight-out, etc.

Firefighters will wait to commence interior structural firefighting, until the proper number of

firefighters can be assembled on scene as required by the response. During this time, the fire

will be attacked only from the outside, sizing-up operations will occur and emergency rescue

necessary to save lives may take place.

One of the standby firefighters may have other duties such as serving as the incident

commander, safety officer, or operator of fire apparatus. However, one of the outside firefighters

must actively monitor the status of the inside firefighters and will not be assigned additional

duties. The second outside firefighter may be involved in a wide variety of activities. Both of the

outside firefighters must be able to provide support and assistance to the two interior firefighters; any assignment of additional duties for one of the outside firefighters must be weighed against the potential for interference with this requirement. Proper assignment of firefighting activities at an interior structural fire must be determined by the incident commander and is dependent on the existing firefighting situation. Consideration of all worksite variables and conditions, and the judgement of the incident commander is critical.

The two firefighters entering an IDLH atmosphere to perform interior structural firefighting must

maintain visual or voice communication at all times. Electronic methods of communication such

as the use of radios shall not be substituted for direct visual contact between team members in

the danger area. However, reliable electronic communication devices are not prohibited and

certainly have value in augmenting communication and may be used to communicate between

inside team members and outside standby firefighters.

SECTION 3 - TRAINING

SELF CONTAINED BREATHING APPARATUS TRAINING

Firefighters wearing respiratory protection shall be trained in proper use, cleaning and

maintenance. No firefighter shall wear respiratory protection without training as specified in this

document.

Training in the use of respiratory protection shall be done in two phases. Each new firefighter

will be given initial training before using respiratory protection and annual training thereafter.

New Recruit Training

Initial training is to be provided during the Fire Fighter I Course at a State approved training

academy. No firefighter is to use respiratory protection unless training has been successfully

completed. Firefighters trained at other than a state approved fire academy must be certified as

trained by the Safety Officer before wearing an SCBA.

Annual Training

On-going training shall be provided to all firefighters of the Department.

Each firefighter must pass a facepiece fit-test during initial and annual training. Appendix B of

this program contains the fit-test protocol and example fit-test record.

Course Content

Initial and annual training in respiratory protection shall be conducted as specified in Appendix

C.

FILL STATION TRAINING

SCBA cylinders will be filled only by firefighters who have completed fill station training.

Retraining will be provided annually.

Course Content

Initial and annual fill station training shall be conducted as specified in Appendix D.

SECTION 4 - RESPIRATOR FITTING AND SEAL CHECK

Each firefighter must pass a facepiece fit-test during initial and annual training. Appendix B of

this program contains an example of a fit-test record.

Inspection Before Use

When using SCBA, each firefighter shall select and wear the correct size facepiece as

determined by initial and annual fit testing. A firefighter shall not wear respiratory protection

unless the proper size facepiece is available and the equipment is in proper working condition

according to the manufacturer’s specifications.

Effective Seal Required

An effective face-to-facepiece seal is extremely important when using respiratory protective

equipment. Minor leakage can allow contaminants to enter the facepiece, even with a positive

pressure SCBA. Any outward leakage will increase the rate of air consumption, reducing the

time available for use and safe exit. The facepiece must seal tightly against the skin, without

penetration or interference by any protective clothing or other equipment.

Nothing can be between the sealing surface of the mask and the face of the wearer, including

but not limited to, eyeglasses, protective hoods, and beards or other facial hair.

Firefighters shall perform a seal check prior to every SCBA use. SCBA can only be worn when

an adequate seal is achieved. (NOTE: the required seal check procedures are found in

Appendix B-1 of the PEOSH Respiratory Protection Standard. The PEOSH Respiratory

Protection Standard is found in Appendix G of this document).

SECTION 5 - INSPECTION, STORAGE, MAINTENANCE AND AIR SUPPLY

Inspection

Regular periodic inspections are required to ensure that all respiratory protection equipment is

properly operating and available for use.

Inspection Schedule

All SCBA and spare cylinders shall be inspected after each use and at least monthly. Guidelines

for inspection are in the manufacturer’s instructions found in Appendix A of this program.

After each inspection, the appropriate forms (see Appendix E) shall be completed. SCBA units

determined to be unfit for use shall be taken out of service, and tagged with a description of the

particular defect.

In the event replacement or repair of SCBA components is necessary, it shall be performed

according to manufacturer’s instructions and only by persons trained and certified by the

manufacturer or returned to the manufacturer’s service facility.

Firefighters will not subject SCBA units to unnecessary abuse due to neglect and/or

carelessness. Caution must especially be exercised to protect the facepiece section of the

mask from being scratched or damaged.

Each SCBA shall be cleaned and disinfected after each use. Only cleaning/sanitizing solutions

for respiratory equipment will be used for cleaning and disinfection. (NOTE: the required SCBA

cleaning procedures are found in Appendix B-2 of the PEOSH Respiratory Protection Standard.

The PEOSH Respiratory Protection Standard is found in Appendix G of this document).

SCBA cylinders shall be hydrostatically tested within the period specified by the manufacturer

and applicable governmental agencies. Metal cylinders must be tested every five (5) years and

composite cylinders every three (3) years. Composite cylinders will be removed from service

after 15 years from the first hydrostatic test date.

Storage

All units shall be stored so that they are protected against direct sunlight, dust accumulation,

severe temperature changes, excessive moisture, fumes, and damaging chemicals. Care is to

be taken so that the means of storage does not distort or damage rubber or elastomeric

components.

Air Supply

Breathing air in the SCBA cylinder shall meet the requirements of the Compressed Gas

Association G-7.1-1989, COMMODITY SPECIFICATION FOR AIR, with a minimum air quality of Grade D. The Fire Department shall ensure that private vendors supplying compressed

breathing air provide a copy of the most recent inspection and certification.

The purity of the air from the Fire Department’s air compressor shall be checked by a competent

laboratory at least annually.

The Fire Department shall assure that sufficient quantities of compressed air are available to

refill SCBA for all emergencies. This shall be accomplished through mutual aid with

__________________ (or: this shall be accomplished with the use of a mobile air compressor).

Air cylinders for SCBA shall be filled only by personnel who have completed fill station training.

Compressed oxygen shall not be used in open-circuit SCBA.

SECTION 6 - MEDICAL EVALUATION

A medical evaluation to determine the firefighter’s ability to wear a SCBA will be provided. Only

firefighters that are medically able to wear SCBA will be allowed to do so. Appendix F contains

the medical evaluation protocol.

SECTION 7 - RECORDKEEPING

Completed SCBA inspection forms will be maintained by ______________________________.

Records/results of air quality tests will be maintained by _______________________________.

Completed fit test records will be maintained by __________________________. Each

firefighter will receive a copy of his/her fit test record.

Records for both recruit training as well as on-going SCBA training records will be maintained by ______________________________.

Certificates of completion for Fire Fighter I courses will be maintained by _________________.

Fill station training records will be maintained by _____________________________.

Medical Evaluation Results Forms will be maintained by _________________________.

SUMMARY OF RESPIRATORY PROTECTION PROGRAM RECORDS

| Type of Record | Keep Records For |
| --- | --- |
| SCBA Inspection Records After Use | until replaced |
| Monthly | one month |
| SCBA Maintenance/Repair Records | life of equipment |
| Air Quality Tests | 1 Year |
| Fit Test 1 Medical Evaluation | Year length of employment , plus thirty years |
| Training | 5 Years |
| Records Documenting Training for Those Who Fill Cylinders | 1 Year |

SECTION 8 - PROGRAM EVALUATION

Evaluation Requirements

The effectiveness of the SCBA program shall be evaluated and corrective actions taken to

ensure the respiratory protection program is properly implemented. The fire department will

regularly consult with firefighters to assess their views on the effectiveness of the program and

to identify any problems.

The evaluation will be conducted by ______________________. The evaluation will ensure:

- procedures for purchasing of approved equipment are in place;

- all firefighters are being properly fitted with respiratory protection;

- all firefighters are properly trained;

- the proper equipment, cleaning, inspection, and maintenance procedures are

implemented;

- the required records are being kept; and

- changes are implemented to correct deficiencies.

Program Monitoring

Periodic monitoring of the respiratory protection program is necessary to ensure that all

firefighters are adequately protected. Random inspections shall be made by the ____________

to ensure that the provisions of the program are being properly implemented.

Appendix A - Manufacturer’s Instructions

NOTE: A copy of the manufacturer’s instructions for the SCBA can be included here or a statement where the instructions are kept and how someone could obtain or borrow a copy.

Appendix B - Respirator Fit Test Record

Date: (of fit test)______________________________________

Firefighter:_______________________________________________________

SCBA Manufacturer:_______________________________________________

Model:____________________________________________________

NIOSH Approval Number:_____________________________________

Facepiece Size

Small_________ Medium__________ Large__________

Conditions which could affect respirator fit:

  Clean Shaven			  Facial Scar 		  Dentures Absent

  1 -2 Day Beard Growth 		  2+ Day Growth

   Moustache 			  Glasses

Comments:_______________________________________________________________

Fit Test Protocol Used _____________________________________________

   Pass 	   Fail

Comments:__________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

Employee Acknowledgment of Test Results:

Employee Name (Print):_____________________________________

Employee Signature_________________________________        Date:________________

Test Conducted By (Print:):________________________________________

(Signature):________________________________________

NOTE: Appendix A of the PEOSH Respiratory Protection Standard contains all the mandatory

fit test protocols. Appendix G of this document contains the PEOSH Respiratory Protection

Standard. One of those protocols must be used.

Appendix C - SCBA Training Outline

At a minimum, the following topics are to be covered in the SCBA training.

1. Why the SCBA is necessary and how improper fit, usage, or maintenance can

compromise the protective effect of the respirator.

2. What the limitations and capabilities of the SCBA are.

3. How to use the SCBA effectively in emergency situations, including situations were the

SCBA malfunctions.

4. Instruction on recognizing medical signs and symptoms that may limit or prevent the

effective use of the SCBA.

5. How to inspect, put on and remove, use, and check the seals of the SCBA.

6. What the procedures are for maintenance, and storage of the SCBA.

7. The general requirements of the PEOSH Respiratory Protection Standard.

APPENDIX D - FILL STATION TRAINING OUTLINE

At a minimum the following topics are to be covered in the fill station training:

1. Procedures for inspecting the SCBA cylinder for damage.

2. Information to ensure that the cylinder has the proper hydrostatic test date.

3. Information to ensure that composite cylinders older than 15 years are not refilled

and are removed from service.

4. Procedures for safely operating the fill station.

5. Information on the importance of using at least grade D air.

6. Information on the consequences of cylinder failure.

7. The manufacturer’s instructions for the fill station.

8. Record keeping requirements.

APPENDIX E - AFTER USE/DAILY INSPECTION FORM

### SCBA Inspection Checklist

Type of Check:     Weekly		 Monthly		 After Use

Checked by:_______________________________________________________

Date:_________________________________

| Regulator No. | R-1 |  | R-2 |  | R-3 |  |  | R-4 |  |  | R-5 |  |  |  | R-6 |  |  |  | R-7 |  |  |  | R-8 |  |  |  | R-9 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bottle No. | B- |  | B- |  | B- |  |  | B- |  |  | B- |  |  |  | B- |  |  |  | B- |  |  |  | B- |  |  |  | B- |  |  |
| Mask No. | M- |  | M- |  | M- |  |  | M- |  |  | M- |  |  |  | M- |  |  |  | M- |  |  |  | M- |  |  |  | M- |  |  |
| ##### Harness Check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conditions of Straps, Buckles, Backplate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| O-Ring in Place |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High Pressure Hose |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Low Pressure Hose |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ##### Operational Check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bottle Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cylinder Pressure (PSI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Harness Gauge Pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressure Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bypass Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pack Alarm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PASS Device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ##### Mask Check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Regulator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exhalation Valve |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cleanliness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### SEE ADDITIONAL COMMENTS ON BACK OF CHECKLIST

COMMENTS

1. _________________________________________________________________________

2. _________________________________________________________________________

3. _________________________________________________________________________

4. _________________________________________________________________________

5. _________________________________________________________________________

6. _________________________________________________________________________

7. _________________________________________________________________________

8. _________________________________________________________________________

9. _________________________________________________________________________

10. _________________________________________________________________________

###### SPARE MASKS

Number Condition

M-

M-

M-

M-

M-

###### SPARE CYLINDERS

Number Condition

APPENDIX F - MEDICAL EVALUATION PROTOCOL

Medical evaluation will be provided to firefighters before they are fit tested for respirator use.

The ABC medical clinic will provide medical evaluations. Medical evaluation procedures are as

follows:

(NOTE: Chose paragraph 1 if firefighters will receive an initial medical examination. Chose

paragraph 2 if firefighters will initially fill out the medical questionnaire.).

paragraph 1 Medical examinations to determine the firefighters ability to wear an SCBA will be

provided by the ABC medical clinic.

paragraph 2 The medical evaluation will be conducted using the required PEOSH

questionnaire. The questionnaire is provided in Appendix C of the PEOSH Respiratory

Protection Standard (the respiratory protection standard is found in Appendix G of this

document). The Program Administrator will provide a copy of this questionnaire to all firefighters

requiring medical evaluations. All affected firefighters will be given a copy of the questionnaire

to fill out, along with a stamped and addressed envelope for mailing the questionnaire to the

ABC medical clinic. Firefighters will be permitted to fill out the questionnaire during work time.

Firefighters will receive follow-up medical evaluations as required by the PEOSH Respiratory

Protection Standard, and/or as deemed necessary by the ABC medical clinic.

Upon request, the firefighter will have the opportunity to speak with the health care professional

about their medical evaluation.

The Program Administrator has provided the ABC medical clinic with a copy of this program, a

copy of the PEOSH Respiratory Protection Standard, information on the type of SCBA used by

the fire department, information on the frequency and length of SCBA use, potential temperature

and humidity extremes, and information on turn-out gear used for firefighting.

Additional medical evaluations will be provided to firefighters under the following circumstances:

- The firefighter reports signs and/or symptoms related to their ability to wear to use an

SCBA, such as shortness of breath, dizziness, chest pains, or wheezing;

- The ABC medical clinic health care provider or supervisor informs the Program

Administrator that the firefighter needs to be reevaluated;

- Information from this program, including observations made during fit testing and

program evaluation, indicates a need for reevaluation.

All examinations and questionnaires are to remain confidential between the firefighter and the

health care provider. All medical records and completed questionnaires will not be kept by the

fire department. The medical records and questionnaires will be under the control of the ABC

medical clinic.

The ABC medical clinic will provide the Program Administrator and firefighter with a written

recommendation regarding the firefighter’s ability to wear a respirator. Only the following

information will be provided:

- a statement on the firefighter’s ability to wear a respirator

- the need for follow-up medical evaluation if any are necessary, and

- a statement that the medical provider has provided the firefighter with a copy of the

recommendation.

Medical records will be maintained in compliance with the PEOSH Access to Employee

Exposure and Medical Records (29CFR1910.1020).

The Fire Department will provide employees access to their medical records. Access means the

right and opportunity to examine and copy records.

APPENDIX G - PEOSH RESPIRATORY PROTECTION STANDARD -29CFR1910.134

(a) Permissible practice.

(a)(1)

In the control of those occupational diseases caused by breathing air contaminated with harmful

dusts, fogs, fumes, mists, gases, smokes, sprays, or vapors, the primary objective shall be to

prevent atmospheric contamination. This shall be accomplished as far as feasible by accepted

engineering control measures (for example, enclosure or confinement of the operation, general

and local ventilation, and substitution of less toxic materials). When effective engineering

controls are not feasible, or while they are being instituted, appropriate respirators shall be used

pursuant to this section.

(a)(2)

Respirators shall be provided by the employer when such equipment is necessary to protect the

health of the employee. The employer shall provide the respirators which are applicable and

suitable for the purpose intended. The employer shall be responsible for the establishment and

maintenance of a respiratory protection program which shall include the requirements outlined in paragraph (c) of this section...1910.134(b).

(b)

Definitions. The following definitions are important terms used in the respiratory protection

standard in this section. Air-purifying respirator means a respirator with an air-purifying filter,

cartridge, or canister that removes specific air contaminants by passing ambient air through the

air-purifying element.

Assigned protection factor (APF) [Reserved]

Atmosphere-supplying respirator means a respirator that supplies the respirator user with

breathing air from a source independent of the ambient atmosphere, and includes supplied-air

respirators (SARs) and self-contained breathing apparatus (SCBA) units.

Canister or cartridge means a container with a filter, sorbent, or catalyst, or combination of these items, which removes specific contaminants from the air passed through the container.

Demand respirator means an atmosphere-supplying respirator that admits breathing air to the

facepiece only when a negative pressure is created inside the facepiece by inhalation.

Emergency situation means any occurrence such as, but not limited to, equipment failure,

rupture of containers, or failure of control equipment that may or does result in an uncontrolled

significant release of an airborne contaminant.

Employee exposure means exposure to a concentration of an airborne contaminant that would

occur if the employee were not using respiratory protection.

End-of-service-life indicator (ESLI) means a system that warns the respirator user of the

approach of the end of adequate respiratory protection, for example, that the sorbent is

approaching saturation or is no longer effective.

Escape-only respirator means a respirator intended to be used only for emergency exit. Filter or

air purifying element means a component used in respirators to remove solid or liquid aerosols

from the inspired air.

Filtering facepiece (dust mask) means a negative pressure particulate respirator with a filter as

an integral part of the facepiece or with the entire facepiece composed of the filtering medium.

Fit factor means a quantitative estimate of the fit of a particular respirator to a specific individual,

and typically estimates the ratio of the concentration of a substance in ambient air to its

concentration inside the respirator when worn.

Fit test means the use of a protocol to qualitatively or quantitatively evaluate the fit of a respirator on an individual. (See also Qualitative fit test QLFT and Quantitative fit test QNFT.)

Helmet means a rigid respiratory inlet covering that also provides head protection against impact and penetration.

High efficiency particulate air (HEPA) filter means a filter that is at least 99.97% efficient in

removing monodisperse particles of 0.3 micrometers in diameter. The equivalent NIOSH 42

CFR 84 particulate filters are the N100, R100, and P100 filters.

Hood means a respiratory inlet covering that completely covers the head and neck and may also cover portions of the shoulders and torso.

Immediately dangerous to life or health (IDLH) means an atmosphere that poses an immediate

threat to life, would cause irreversible adverse health effects, or would impair an individual's

ability to escape from a dangerous atmosphere.

Interior structural firefighting means the physical activity of fire suppression, rescue or both,

inside of buildings or enclosed structures which are involved in a fire situation beyond the

incipient stage. (See N.J.A.C.12:100-10)

Loose-fitting facepiece means a respiratory inlet covering that is designed to form a partial seal

with the face.

Maximum use concentration (MUC) [Reserved].

Negative pressure respirator (tight fitting) means a respirator in which the air pressure inside the

facepiece is negative during inhalation with respect to the ambient air pressure outside the

respirator.

Oxygen deficient atmosphere means an atmosphere with an oxygen content below 19.5% by

volume.

Physician or other licensed health care professional (PLHCP) means an individual whose legally

permitted scope of practice (i.e., license, registration, or certification) allows him or her to

independently provide, or be delegated the responsibility to provide, some or all of the health

care services required by paragraph (e) of this section.

Positive pressure respirator means a respirator in which the pressure inside the respiratory inlet

covering exceeds the ambient air pressure outside the respirator.

Powered air-purifying respirator (PAPR) means an air-purifying respirator that uses a blower to

force the ambient air through air-purifying elements to the inlet covering.

Pressure demand respirator means a positive pressure atmosphere-supplying respirator that

admits breathing air to the facepiece when the positive pressure is reduced inside the facepiece

by inhalation.

Qualitative fit test (QLFT) means a pass/fail fit test to assess the adequacy of respirator fit that

relies on the individual's response to the test agent.

Quantitative fit test (QNFT) means an assessment of the adequacy of respirator fit by

numerically measuring the amount of leakage into the respirator.

Respiratory inlet covering means that portion of a respirator that forms the protective barrier

between the user's respiratory tract and an air-purifying device or breathing air source, or both. It may be a facepiece, helmet, hood, suit, or a mouthpiece respirator with nose clamp.

Self-contained breathing apparatus (SCBA) means an atmosphere-supplying respirator for

which the breathing air source is designed to be carried by the user.

Service life means the period of time that a respirator, filter or sorbent, or other respiratory

equipment provides adequate protection to the wearer.

Supplied-air respirator (SAR) or airline respirator means an atmosphere-supplying respirator for

which the source of breathing air is not designed to be carried by the user. This section means

this respiratory protection standard.

Tight-fitting facepiece means a respiratory inlet covering that forms a complete seal with the

face.

User seal check means an action conducted by the respirator user to determine if the respirator

is properly seated to the face.

(c)

Respiratory protection program. This paragraph requires the employer to develop and implement a written respiratory protection program with required worksite-specific procedures and elements for required respirator use. The program must be administered by a suitably trained program administrator. In addition, certain program elements may be required for voluntary use to prevent potential hazards associated with the use of the respirator. The Small Entity Compliance Guide contains criteria for the selection of a program administrator and a sample program that meets the requirements of this paragraph. Copies of the Small Entity Compliance Guide will be available on or about April 8, 1998 from the Occupational Safety and Health Administration's Office of Publications, Room N 3101, 200 Constitution Avenue, NW, Washington, DC, 20210 (202-219-4667).

(c)(1)

In any workplace where respirators are necessary to protect the health of the employee or

whenever respirators are required by the employer, the employer shall establish and implement

a written respiratory protection program with worksite-specific procedures. The program shall

be updated as necessary to reflect those changes in workplace conditions that affect respirator

use. The employer shall include in the program the following provisions of this section, as

applicable:

(c)(1)(i)

Procedures for selecting respirators for use in the workplace;

(c)(1)(ii)

Medical evaluations of employees required to use respirators;

(c)(1)(iii)

Fit testing procedures for tight-fitting respirators;

(c)(1)(iv)

Procedures for proper use of respirators in routine and reasonably foreseeable emergency

situations;

(c)(1)(v)

Procedures and schedules for cleaning, disinfecting, storing, inspecting, repairing, discarding,

and otherwise maintaining respirators;

(c)(1)(vi)

Procedures to ensure adequate air quality, quantity, and flow of breathing air for

atmosphere-supplying respirators;

(c)(1)(vii)

Training of employees in the respiratory hazards to which they are potentially exposed during

routine and emergency situations;

(c)(1)(viii)

Training of employees in the proper use of respirators, including putting on and removing them,

any limitations on their use, and their maintenance; and

(c)(1)(ix)

Procedures for regularly evaluating the effectiveness of the program.

(c)(2)

Where respirator use is not required:

(c)(2)(i)

An employer may provide respirators at the request of employees or permit employees to use

their own respirators, if the employer determines that such respirator use will not in itself create a hazard. If the employer determines that any voluntary respirator use is permissible, the employer shall provide the respirator users with the information contained in Appendix D to this section ("Information for Employees Using Respirators When Not Required Under the Standard"); and

(c)(2)(ii)

In addition, the employer must establish and implement those elements of a written respiratory

protection program necessary to ensure that any employee using a respirator voluntarily is

medically able to use that respirator, and that the respirator is cleaned, stored, and maintained

so that its use does not present a health hazard to the user. Exception: Employers are not

required to include in a written respiratory protection program those employees whose only use

of respirators involves the voluntary use of filtering facepieces (dust masks).

(c)(3)

The employer shall designate a program administrator who is qualified by appropriate training or

experience that is commensurate with the complexity of the program to administer or oversee

the respiratory protection program and conduct the required evaluations of program

effectiveness.

(c)(4)

The employer shall provide respirators, training, and medical evaluations at no cost to the

employee.

(d)

Selection of respirators. This paragraph requires the employer to evaluate respiratory hazard(s)

in the workplace, identify relevant workplace and user factors, and base respirator selection on

these factors. The paragraph also specifies appropriately protective respirators for use in IDLH

atmospheres, and limits the selection and use of air-purifying respirators.

(d)(1)

General requirements.

(d)(1)(i)

The employer shall select and provide an appropriate respirator based on the respiratory

hazard(s) to which the worker is exposed and workplace and user factors that affect respirator

performance and reliability.

(d)(1)(ii)

The employer shall select a NIOSH-certified respirator. The respirator shall be used in

compliance with the conditions of its certification.

(d)(1)(iii)

The employer shall identify and evaluate the respiratory hazard(s) in the workplace; this

evaluation shall include a reasonable estimate of employee exposures to respiratory hazard(s)

and an identification of the contaminant's chemical state and physical form. Where the employer

cannot identify or reasonably estimate the employee exposure, the employer shall consider the

atmosphere to be IDLH.

(d)(1)(iv)

The employer shall select respirators from a sufficient number of respirator models and sizes so

that the respirator is acceptable to, and correctly fits, the user.

(d)(2)

Respirators for IDLH atmospheres.

(d)(2)(i)

The employer shall provide the following respirators for employee use in IDLH atmospheres:

(d)(2)(i)(A)

A full facepiece pressure demand SCBA certified by NIOSH for a minimum service life of thirty

minutes, or

(d)(2)(i)(B)

A combination full facepiece pressure demand supplied-air respirator (SAR) with auxiliary

self-contained air supply.

(d)(2)(ii)

Respirators provided only for escape from IDLH atmospheres shall be NIOSH-certified for

escape from the atmosphere in which they will be used.

(d)(2)(iii)

All oxygen-deficient atmospheres shall be considered IDLH. Exception: If the employer

demonstrates that, under all foreseeable conditions, the oxygen concentration can be

maintained within the ranges specified in Table II of this section (i.e., for the altitudes set out in

the table), then any atmosphere-supplying respirator may be used.

(d)(3)

Respirators for atmospheres that are not IDLH.

(d)(3)(i)

The employer shall provide a respirator that is adequate to protect the health of the employee

and ensure compliance with all other OSHA statutory and regulatory requirements, under routine and reasonably foreseeable emergency situations.

(d)(3)(i)(A)

Assigned Protection Factors (APFs) [Reserved]

(d)(3)(i)(B)

Maximum Use Concentration (MUC) [Reserved]

(d)(3)(ii)

The respirator selected shall be appropriate for the chemical state and physical form of the

contaminant.

(d)(3)(iii)

For protection against gases and vapors, the employer shall provide:

(d)(3)(iii)(A)

An atmosphere-supplying respirator, or

(d)(3)(iii)(B)

An air-purifying respirator, provided that:

(d)(3)(iii)(B)(1)

(1) The respirator is equipped with an end-of-service-life indicator (ESLI) certified by NIOSH for

the contaminant; or

(d)(3)(iii)(B)(2)

If there is no ESLI appropriate for conditions in the employer's workplace, the employer

implements a change schedule for canisters and cartridges that is based on objective

information or data that will ensure that canisters and cartridges are changed before the end of

their service life. The employer shall describe in the respirator program the information and data

relied upon and the basis for the canister and cartridge change schedule and the basis for

reliance on the data.

(d)(3)(iv)

For protection against particulates, the employer shall provide:

(d)(3)(iv)(A)

An atmosphere-supplying respirator; or

(d)(3)(iv)(B)

An air-purifying respirator equipped with a filter certified by NIOSH under 30 CFR part 11 as a

high efficiency particulate air (HEPA) filter, or an air-purifying respirator equipped with a filter

certified for particulates by NIOSH under 42 CFR part 84; or

(d)(3)(iv)(C)

For contaminants consisting primarily of particles with mass median aerodynamic diameters

(MMAD) of at least 2 micrometers, an air-purifying respirator equipped with any filter certified for

particulates by NIOSH.

TABLE I. -- Assigned Protection

Factors [Reserved]

### TABLE II

| Altitude (ft.) | Oxygen deficient atmospheres (%O ) for which the employer may rely on atmosphere-supplying respirators |
| --- | --- |
| Less than 3001 | 16.0 - 19.5 |
| 3001 - 4000 | 16.4 - 19.5 |
| 4001 - 5000 | 17.1 - 19.5 |
| 5001 - 6000 | 17.8 - 19.5 |
| 6001 – 7000 | 18.5 - 19.5 |
| 7001 - 8000 | 19.3 - 19.5 |

Above 8000 feet the exception does not apply. Oxygen- enriched breathing air must be supplied above 14000 feet.

1

(e)

Medical evaluation. Using a respirator may place a physiological burden on employees that

varies with the type of respirator worn, the job and workplace conditions in which the respirator is used, and the medical status of the employee. Accordingly, this paragraph specifies the

minimum requirements for medical evaluation that employers must implement to determine the

employee's ability to use a respirator.

(e)(1)

General. The employer shall provide a medical evaluation to determine the employee's ability to

use a respirator, before the employee is fit tested or required to use the respirator in the

workplace. The employer may discontinue an employee's medical evaluations when the

employee is no longer required to use a respirator.

(e)(2)

Medical evaluation procedures.

(e)(2)(i)

The employer shall identify a physician or other licensed health care professional (PLHCP) to

perform medical evaluations using a medical questionnaire or an initial medical examination that

obtains the same information as the medical questionnaire.

(e)(2)(ii)

The medical evaluation shall obtain the information requested by the questionnaire in Sections 1

and 2, Part A of Appendix C of this section.

(e)(3)

Follow-up medical examination.

(e)(3)(i)

The employer shall ensure that a follow-up medical examination is provided for an employee

who gives a positive response to any question among questions 1 through 8 in Section 2, Part A

of Appendix C or whose initial medical examination demonstrates the need for a follow-up

medical examination.

(e)(3)(ii)

The follow-up medical examination shall include any medical tests, consultations, or diagnostic

procedures that the PLHCP deems necessary to make a final determination.

(e)(4)

Administration of the medical questionnaire and examinations.

(e)(4)(i)

The medical questionnaire and examinations shall be administered confidentially during the

employee's normal working hours or at a time and place convenient to the employee. The

medical questionnaire shall be administered in a manner that ensures that the employee

understands its content.

(e)(4)(ii)

The employer shall provide the employee with an opportunity to discuss the questionnaire and

examination results with the PLHCP.

(e)(5)

Supplemental information for the PLHCP.

(e)(5)(i)

The following information must be provided to the PLHCP before the PLHCP makes a

recommendation concerning an employee's ability to use a respirator:

(e)(5)(i)(A)

(A) The type and weight of the respirator to be used by the employee;

(e)(5)(i)(B)

The duration and frequency of respirator use (including use for rescue and escape);

(e)(5)(i)(C)

The expected physical work effort;

(e)(5)(i)(D)

Additional protective clothing and equipment to be worn; and

(e)(5)(i)(E)

Temperature and humidity extremes that may be encountered.

(e)(5)(ii)

Any supplemental information provided previously to the PLHCP regarding an employee need

not be provided for a subsequent medical evaluation if the information and the PLHCP remain

the same.

(e)(5)(iii)

The employer shall provide the PLHCP with a copy of the written respiratory protection program

and a copy of this section.

Note to Paragraph (e)(5)(iii): When the employer replaces a PLHCP, the employer must ensure

that the new PLHCP obtains this information, either by providing the documents directly to the

PLHCP or having the documents transferred from the former PLHCP to the new PLHCP.

However, OSHA does not expect employers to have employees medically reevaluated solely

because a new PLHCP has been selected.

(e)(6)

Medical determination. In determining the employee's ability to use a respirator, the employer

shall:

(e)(6)(i)

Obtain a written recommendation regarding the employee's ability to use the respirator from the

PLHCP. The recommendation shall provide only the following information:

(e)(6)(i)(A)

Any limitations on respirator use related to the medical condition of the employee, or relating to

the workplace conditions in which the respirator will be used, including whether or not the

employee is medically able to use the respirator;

(e)(6)(i)(B)

The need, if any, for follow-up medical evaluations; and

(e)(6)(i)(C)

A statement that the PLHCP has provided the employee with a copy of the PLHCP's written

recommendation.

(e)(6)(ii)

If the respirator is a negative pressure respirator and the PLHCP finds a medical condition that

may place the employee's health at increased risk if the respirator is used, the employer shall

provide a PAPR if the PLHCP's medical evaluation finds that the employee can use such a

respirator; if a subsequent medical evaluation finds that the employee is medically able to use a

negative pressure respirator, then the employer is no longer required to provide a PAPR.

(e)(7)

Additional medical evaluations. At a minimum, the employer shall provide additional medical

evaluations that comply with the requirements of this section if:

(e)(7)(i)

An employee reports medical signs or symptoms that are related to ability to use a respirator;

(e)(7)(ii)

A PLHCP, supervisor, or the respirator program administrator informs the employer that an

employee needs to be reevaluated;

(e)(7)(iii)

Information from the respiratory protection program, including observations made during fit

testing and program evaluation, indicates a need for employee reevaluation; or

(e)(7)(iv)

A change occurs in workplace conditions (e.g., physical work effort, protective clothing,

temperature) that may result in a substantial increase in the physiological burden placed on an

employee.

(f)

Fit testing. This paragraph requires that, before an employee may be required to use any

respirator with a negative or positive pressure tight-fitting facepiece, the employee must be fit

tested with the same make, model, style, and size of respirator that will be used. This paragraph

specifies the kinds of fit tests allowed, the procedures for conducting them, and how the results

of the fit tests must be used.

(f)(1)

The employer shall ensure that employees using a tight-fitting facepiece respirator pass an

appropriate qualitative fit test (QLFT) or quantitative fit test (QNFT) as stated in this paragraph.

(f)(2)

The employer shall ensure that an employee using a tight-fitting facepiece respirator is fit tested

prior to initial use of the respirator, whenever a different respirator facepiece (size, style, model

or make) is used, and at least annually thereafter.

(f)(3)

The employer shall conduct an additional fit test whenever the employee reports, or the

employer, PLHCP, supervisor, or program administrator makes visual observations of, changes

in the employee's physical condition that could affect respirator fit. Such conditions include, but

are not limited to, facial scarring, dental changes, cosmetic surgery, or an obvious change in

body weight.

(f)(4)

If after passing a QLFT or QNFT, the employee subsequently notifies the employer, program

administrator, supervisor, or PLHCP that the fit of the respirator is unacceptable, the employee

shall be given a reasonable opportunity to select a different respirator facepiece and to be

retested.

(f)(5)

The fit test shall be administered using an OSHA-accepted QLFT or QNFT protocol. The

OSHA-accepted QLFT and QNFT protocols and procedures are contained in Appendix A of this

section.

(f)(6)

QLFT may only be used to fit test negative pressure air-purifying respirators that must achieve a

fit factor of 100 or less.

(f)(7)

If the fit factor, as determined through an OSHA-accepted QNFT protocol, is equal to or greater

than 100 for tight-fitting half facepieces, or equal to or greater than 500 for tight-fitting full

facepieces, the QNFT has been passed with that respirator.

(f)(8)

Fit testing of tight-fitting atmosphere-supplying respirators and tight-fitting powered air-purifying

respirators shall be accomplished by performing quantitative or qualitative fit testing in the

negative pressure mode, regardless of the mode of operation (negative or positive pressure)

that is used for respiratory protection.

(f)(1)(8)(i)

Qualitative fit testing of these respirators shall be accomplished by temporarily converting the

respirator user's actual facepiece into a negative pressure respirator with appropriate filters, or

by using an identical negative pressure air-purifying respirator facepiece with the same sealing

surfaces as a surrogate for the atmosphere-supplying or powered air-purifying respirator

facepiece.

(f)(1)(8)(ii)

Quantitative fit testing of these respirators shall be accomplished by modifying the facepiece to

allow sampling inside the facepiece in the breathing zone of the user, midway between the nose

and mouth. This requirement shall be accomplished by installing a permanent sampling probe

onto a surrogate facepiece, or by using a sampling adapter designed to temporarily provide a

means of sampling air from inside the facepiece.

(f)(1)(8)(iii)

Any modifications to the respirator facepiece for fit testing shall be completely removed, and the

facepiece restored to NIOSH-approved configuration, before that facepiece can be used in the

workplace.

(g)

Use of respirators. This paragraph requires employers to establish and implement procedures

for the proper use of respirators. These requirements include prohibiting conditions that may

result in facepiece seal leakage, preventing employees from removing respirators in hazardous

environments, taking actions to ensure continued effective respirator operation throughout the

work shift, and establishing procedures for the use of respirators in IDLH atmospheres or in

interior structural firefighting situations.

(g)(1)

Facepiece seal protection.

(g)(1)(i)

The employer shall not permit respirators with tight-fitting facepieces to be worn by employees

who have:

(g)(1)(i)(A)

Facial hair that comes between the sealing surface of the facepiece and the face or that

interferes with valve function; or

(g)(1)(i)(B)

Any condition that interferes with the face-to-facepiece seal or valve function.

(g)(1)(ii)

If an employee wears corrective glasses or goggles or other personal protective equipment, the

employer shall ensure that such equipment is worn in a manner that does not interfere with the

seal of the facepiece to the face of the user.

(g)(1)(iii)

For all tight-fitting respirators, the employer shall ensure that employees perform a user seal

check each time they put on the respirator using the procedures in Appendix B-1 or procedures

recommended by the respirator manufacturer that the employer demonstrates are as effective

as those in Appendix B-1 of this section.

(g)(2)

Continuing respirator effectiveness.

(g)(2)(i)

Appropriate surveillance shall be maintained of work area conditions and degree of employee

exposure or stress. When there is a change in work area conditions or degree of employee

exposure or stress that may affect respirator effectiveness, the employer shall reevaluate the

continued effectiveness of the respirator.

(g)(2)(ii)

The employer shall ensure that employees leave the respirator use area:

(g)(2)(ii)(A)

To wash their faces and respirator facepieces as necessary to prevent eye or skin irritation

associated with respirator use; or

(g)(2)(ii)(B)

If they detect vapor or gas breakthrough, changes in breathing resistance, or leakage of the

facepiece; or

(g)(2)(ii)(C)

To replace the respirator or the filter, cartridge, or canister elements.

(g)(2)(iii)

If the employee detects vapor or gas breakthrough, changes in breathing resistance, or leakage

of the facepiece, the employer must replace or repair the respirator before allowing the

employee to return to the work area.

(g)(3)

Procedures for IDLH atmospheres. For all IDLH atmospheres, the employer shall ensure that:

(g)(3)(i)

One employee or, when needed, more than one employee is located outside the IDLH

atmosphere;

(g)(3)(ii)

Visual, voice, or signal line communication is maintained between the employee(s) in the IDLH

atmosphere and the employee(s) located outside the IDLH atmosphere;

(g)(3)(iii)

The employee(s) located outside the IDLH atmosphere are trained and equipped to provide

effective emergency rescue;

(g)(3)(iv)

The employer or designee is notified before the employee(s) located outside the IDLH

atmosphere enter the IDLH atmosphere to provide emergency rescue;

(g)(3)(v)

The employer or designee authorized to do so by the employer, once notified, provides

necessary assistance appropriate to the situation;

(g)(3)(vi)

Employee(s) located outside the IDLH atmospheres are equipped with:

(g)(3)(vi)(A)

Pressure demand or other positive pressure SCBAs, or a pressure demand or other positive

pressure supplied-air respirator with auxiliary SCBA; and either

(g)(3)(vi)(B)

Appropriate retrieval equipment for removing the employee(s) who enter(s) these hazardous

atmospheres where retrieval equipment would contribute to the rescue of the employee(s) and

would not increase the overall risk resulting from entry; or

(g)(3)(vi)(C)

Equivalent means for rescue where retrieval equipment is not required under paragraph

(g)(3)(vi)(B).

(g)(4)

Procedures for interior structural firefighting. In addition to the requirements set forth under

paragraph (g)(3), in interior structural fires, the employer shall ensure that:

(g)(4)(i)

At least two employees enter the IDLH atmosphere and remain in visual or voice contact with

one another at all times;

(g)(4)(ii)

At least two employees are located outside the IDLH atmosphere; and

(g)(4)(iii)

All employees engaged in interior structural firefighting use SCBAs.

Note 1 to paragraph (g): One of the two individuals located outside the IDLH atmosphere may

be assigned to an additional role, such as incident commander in charge of the emergency or

safety officer, so long as this individual is able to perform assistance or rescue activities without

jeopardizing the safety or health of any firefighter working at the incident.

Note 2 to paragraph (g): Nothing in this section is meant to preclude firefighters from

performing emergency rescue activities before an entire team has assembled.

(h)

Maintenance and care of respirators. This paragraph requires the employer to provide for the

cleaning and disinfecting, storage, inspection, and repair of respirators used by employees.

(h)(1)

Cleaning and disinfecting. The employer shall provide each respirator user with a respirator that

is clean, sanitary, and in good working order. The employer shall ensure that respirators are

cleaned and disinfected using the procedures in Appendix B-2 of this section, or procedures

recommended by the respirator manufacturer, provided that such procedures are of equivalent

effectiveness. The respirators shall be cleaned and disinfected at the following intervals:

(h)(1)(i)

Respirators issued for the exclusive use of an employee shall be cleaned and disinfected as

often as necessary to be maintained in a sanitary condition;

(h)(1)(ii)

Respirators issued to more than one employee shall be cleaned and disinfected before being

worn by different individuals;

(h)(1)(iii)

Respirators maintained for emergency use shall be cleaned and disinfected after each use; and

(h)(1)(iv)

Respirators used in fit testing and training shall be cleaned and disinfected after each use.

(h)(2)

Storage. The employer shall ensure that respirators are stored as follows:

(h)(2)(i)

All respirators shall be stored to protect them from damage, contamination, dust, sunlight,

extreme temperatures, excessive moisture, and damaging chemicals, and they shall be packed

or stored to prevent deformation of the facepiece and exhalation valve.

(h)(2)(ii)

In addition to the requirements of paragraph (h)(2)(i) of this section, emergency respirators shall

be:

(h)(2)(ii)(A)

Kept accessible to the work area;

(h)(2)(ii)(B)

Stored in compartments or in covers that are clearly marked as containing emergency

respirators; and

(h)(2)(ii)(C)

Stored in accordance with any applicable manufacturer instructions.

(h)(3)

Inspection.

(h)(3)(i)

The employer shall ensure that respirators are inspected as follows:

(h)(3)(i)(A)

All respirators used in routine situations shall be inspected before each use and during cleaning;

(h)(3)(i)(B)

All respirators maintained for use in emergency situations shall be inspected at least monthly

and in accordance with the manufacturer's recommendations, and shall be checked for proper

function before and after each use; and

(h)(3)(i)(C)

Emergency escape-only respirators shall be inspected before being carried into the workplace

for use.

(h)(3)(ii)

The employer shall ensure that respirator inspections include the following:

(h)(3)(ii)(A)

A check of respirator function, tightness of connections, and the condition of the various parts

including, but not limited to, the facepiece, head straps, valves, connecting tube, and cartridges,

canisters or filters; and

(h)(3)(ii)(B)

A check of elastomeric parts for pliability and signs of deterioration.

(h)(3)(iii)

In addition to the requirements of paragraphs (h)(3)(i) and (ii) of this section, self-contained

breathing apparatus shall be inspected monthly. Air and oxygen cylinders shall be maintained in

a fully charged state and shall be recharged when the pressure falls to 90% of the

manufacturer's recommended pressure level. The employer shall determine that the regulator

and warning devices function properly.

(h)(3)(iv)

For respirators maintained for emergency use, the employer shall:

(h)(3)(iv)(A)

Certify the respirator by documenting the date the inspection was performed, the name (or

signature) of the person who made the inspection, the findings, required remedial action, and a

serial number or other means of identifying the inspected respirator; and (B) Provide this

information on a tag or label that is attached to the storage compartment for the respirator, is

kept with the respirator, or is included in inspection reports stored as paper or electronic files.

This information shall be maintained until replaced following a subsequent certification.

(h)(4)

Repairs. The employer shall ensure that respirators that fail an inspection or are otherwise found to be defective are removed from service, and are discarded or repaired or adjusted in

accordance with the following procedures:

(h)(4)(i)

Repairs or adjustments to respirators are to be made only by persons appropriately trained to

perform such operations and shall use only the respirator manufacturer's NIOSH-approved parts

designed for the respirator;

(h)(4)(ii)

Repairs shall be made according to the manufacturer's recommendations and specifications for

the type and extent of repairs to be performed; and

(h)(4)(iii)

Reducing and admission valves, regulators, and alarms shall be adjusted or repaired only by the manufacturer or a technician trained by the manufacturer.

(i)

Breathing air quality and use. This paragraph requires the employer to provide employees using

atmosphere-supplying respirators (supplied-air and SCBA) with breathing gases of high purity.

(i)(1)

The employer shall ensure that compressed air, compressed oxygen, liquid air, and liquid

oxygen used for respiration accords with the following specifications:

(i)(1)(i)

Compressed and liquid oxygen shall meet the United States Pharmacopoeia

requirements for medical or breathing oxygen; and

1910.134(i)(1)(ii)

(i)(1)(ii)

Compressed breathing air shall meet at least the requirements for Grade D breathing air

described in ANSI/Compressed Gas Association Commodity Specification for Air, G-7.1-1989, to include:

(i)(1)(ii)(A)

Oxygen content (v/v) of 19.5-23.5%;

(i)(1)(ii)(B)

Hydrocarbon (condensed) content of 5 milligrams per cubic meter of air or less;

(i)(1)(ii)(C)

Carbon monoxide (CO) content of 10 ppm or less;

(i)(1)(ii)(D)

Carbon dioxide content of 1,000 ppm or less; and

(i)(1)(ii)(E)

Lack of noticeable odor.

(i)(2)

The employer shall ensure that compressed oxygen is not used in atmosphere-supplying

respirators that have previously used compressed air.

(i)(3)

The employer shall ensure that oxygen concentrations greater than 23.5% are used only in

equipment designed for oxygen service or distribution.

(i)(4)

The employer shall ensure that cylinders used to supply breathing air to respirators meet the

following requirements:

(i)(4)(i)

Cylinders are tested and maintained as prescribed in the Shipping Container Specification

Regulations of the Department of Transportation (49 CFR part 173 and part 178);

(i)(4)(ii)

Cylinders of purchased breathing air have a certificate of analysis from the supplier that the

breathing air meets the requirements for Grade D breathing air; and

(i)(4)(iii)

The moisture content in the cylinder does not exceed a dew point of -50 deg.F (-45.6 deg.C) at 1 atmosphere pressure.

(i)(5)

The employer shall ensure that compressors used to supply breathing air to respirators are

constructed and situated so as to:

(i)(5)(i)

Prevent entry of contaminated air into the air-supply system;

(i)(5)(ii)

Minimize moisture content so that the dew point at 1 atmosphere pressure is 10 degrees F (5.56

deg.C) below the ambient temperature;

(i)(5)(iii)

Have suitable in-line air-purifying sorbent beds and filters to further ensure breathing air quality.

Sorbent beds and filters shall be maintained and replaced or refurbished periodically following

the manufacturer's instructions.

(i)(5)(iv)

Have a tag containing the most recent change date and the signature of the person authorized

by the employer to perform the change. The tag shall be maintained at the compressor.

(i)(6)

For compressors that are not oil-lubricated, the employer shall ensure that carbon monoxide

levels in the breathing air do not exceed 10 ppm.

(i)(7)

For oil-lubricated compressors, the employer shall use a high-temperature or carbon monoxide

alarm, or both, to monitor carbon monoxide levels. If only high-temperature alarms are used, the

air supply shall be monitored at intervals sufficient to prevent carbon monoxide in the breathing

air from exceeding 10 ppm.

(i)(8)

The employer shall ensure that breathing air couplings are incompatible with outlets for

nonrespirable worksite air or other gas systems. No asphyxiating substance shall be introduced

into breathing air lines.

(i)(9)

The employer shall use breathing gas containers marked in accordance with the NIOSH

respirator certification standard, 42 CFR part 84.

(j)

Identification of filters, cartridges, and canisters. The employer shall ensure that all filters,

cartridges and canisters used in the workplace are labeled and color coded with the NIOSH

approval label and that the label is not removed and remains legible.

(k)

Training and information. This paragraph requires the employer to provide effective training to

employees who are required to use respirators. The training must be comprehensive,

understandable, and recur annually, and more often if necessary. This paragraph also requires

the employer to provide the basic information on respirators in Appendix D of this section to

employees who wear respirators when not required by this section or by the employer to do so.

(k)(1)

The employer shall ensure that each employee can demonstrate knowledge of at least the

following:

(k)(1)(i)

Why the respirator is necessary and how improper fit, usage, or maintenance can compromise

the protective effect of the respirator;

(k)(1)(ii)

What the limitations and capabilities of the respirator are;

(k)(1)(iii)

How to use the respirator effectively in emergency situations, including situations in which the

respirator malfunctions;

(k)(1)(iv)

How to inspect, put on and remove, use, and check the seals of the respirator;

(k)(1)(v)

What the procedures are for maintenance and storage of the respirator;

(k)(1)(vi)

How to recognize medical signs and symptoms that may limit or prevent the effective use of

respirators; and

(k)(1)(vii)

The general requirements of this section.

(k)(2)

The training shall be conducted in a manner that is understandable to the employee.

(k)(3)

The employer shall provide the training prior to requiring the employee to use a respirator in the

workplace.

(k)(4)

An employer who is able to demonstrate that a new employee has received training within the

last 12 months that addresses the elements specified in paragraph (k)(1)(i) through (vii) is not

required to repeat such training provided that, as required by paragraph (k)(1), the employee

can demonstrate knowledge of those element(s). Previous training not repeated initially by the

employer must be provided no later than 12 months from the date of the previous training.

(k)(5)

Retraining shall be administered annually, and when the following situations occur:

(k)(5)(i)

Changes in the workplace or the type of respirator render previous training obsolete;

(k)(5)(ii)

Inadequacies in the employee's knowledge or use of the respirator indicate that the employee

has not retained the requisite understanding or skill; or

(k)(5)(iii)

Any other situation arises in which retraining appears necessary to ensure safe respirator use.

(k)(6)

The basic advisory information on respirators, as presented in Appendix D of this section, shall

be provided by the employer in any written or oral format, to employees who wear respirators

when such use is not required by this section or by the employer.

(l)

Program evaluation. This section requires the employer to conduct evaluations of the workplace

to ensure that the written respiratory protection program is being properly implemented, and to

consult employees to ensure that they are using the respirators properly.

(l)(1)

The employer shall conduct evaluations of the workplace as necessary to ensure that the

provisions of the current written program are being effectively implemented and that it continues

to be effective.

(l)(2)

The employer shall regularly consult employees required to use respirators to assess the

employees' views on program effectiveness and to identify any problems. Any problems that are

identified during this assessment shall be corrected. Factors to be assessed include, but are

not limited to:

(l)(2)(i)

Respirator fit (including the ability to use the respirator without interfering with effective

workplace performance);

(l)(2)(ii)

Appropriate respirator selection for the hazards to which the employee is exposed;

(l)(2)(iii)

Proper respirator use under the workplace conditions the employee encounters; and

(l)(2)(iv)

Proper respirator maintenance.

(m)

Recordkeeping. This section requires the employer to establish and retain written information

regarding medical evaluations, fit testing, and the respirator program. This information will

facilitate employee involvement in the respirator program, assist the employer in auditing the

adequacy of the program, and provide a record for compliance determinations by OSHA..

(m)(1)

Medical evaluation. Records of medical evaluations required by this section must be retained

and made available in accordance with 29 CFR 1910.1020.

(m)(2)

Fit testing.

(m)(2)(i)

The employer shall establish a record of the qualitative and quantitative fit tests administered to

an employee including:

(m)(2)(i)(A)

The name or identification of the employee tested;

(m)(2)(i)(B)

Type of fit test performed;

(m)(2)(i)(C)

Specific make, model, style, and size of respirator tested;

(m)(2)(i)(D)

Date of test; and

(m)(2)(i)(E)

The pass/fail results for QLFTs or the fit factor and strip chart recording or other recording of the

test results for QNFTs.

(m)(2)(ii)

Fit test records shall be retained for respirator users until the next fit test is administered.

(m)(3)

A written copy of the current respirator program shall be retained by the employer.

(m)(4)

Written materials required to be retained under this paragraph shall be made available upon

request to affected employees and to the Assistant Secretary or designee for examination and

copying.

(n)

Dates.

(n)(1)

Effective date. This section is effective September 21, 1998.

(n)(2)

Operative date. All obligations of this section commence on March 21, 1999.

(n)(3)

The provisions of 29 CFR 1910.134 and 29 CFR 1926.103, contained in the 29 CFR parts 1900

to 1910.99 and the 29 CFR part 1926 editions, revised as of July 1, 1997, are in effect and

enforceable until March 21, 1999, or during any administrative or judicial stay of the provisions of this section.

(n)(4)

Existing Respiratory Protection Programs. If, in the 12 month period preceding September 21,

1998, the employer has conducted annual respirator training, fit testing, respirator program

evaluation, or medical evaluations, the employer may use the results of those activities to

comply with the corresponding provisions of this section, providing that these activities were

conducted in a manner that meets the requirements of this section.

(o)

Appendices.

(o)(1)

Compliance with Appendix A, Appendix B-1, Appendix B-2, and Appendix C

of this section is mandatory.

(o)(2)

Appendix D of this section is non-mandatory and is not intended to

create any additional obligations not otherwise imposed or to detract

from any existing obligations.

Appendix A: Fit Testing Procedures (Mandatory)

Part I. OSHA-Accepted Fit Test Protocols

A. Fit Testing Procedures -- General Requirements

The employer shall conduct fit testing using the following procedures. The requirements in this

appendix apply to all OSHA-accepted fit test methods, both QLFT and QNFT.

1. The test subject shall be allowed to pick the most acceptable respirator from a sufficient

number of respirator models and sizes so that the respirator is acceptable to, and correctly fits,

the user.

2. Prior to the selection process, the test subject shall be shown how to put on a respirator, how

it should be positioned on the face, how to set strap tension and how to determine an acceptable fit. A mirror shall be available to assist the subject in evaluating the fit and positioning of the respirator. This instruction may not constitute the subject's formal training on respirator use, because it is only a review.

3. The test subject shall be informed that he/she is being asked to select the respirator that

provides the most acceptable fit. Each respirator represents a different size and shape, and if

fitted and used properly, will provide adequate protection.

4. The test subject shall be instructed to hold each chosen facepiece up to the face and

eliminate those that obviously do not give an acceptable fit.

5. The more acceptable facepieces are noted in case the one selected proves unacceptable; the most comfortable mask is donned and worn at least five minutes to assess comfort. Assistance in assessing comfort can be given by discussing the points in the following item A.6. If the test subject is not familiar with using a particular respirator, the test subject shall be directed to don the mask several times and to adjust the straps each time to become adept at setting proper tension on the straps.

6. Assessment of comfort shall include a review of the following points with the test subject and

allowing the test subject adequate time to determine the comfort of the respirator:

(a) Position of the mask on the nose

(b) Room for eye protection

(c) Room to talk

(d) Position of mask on face and cheeks

7. The following criteria shall be used to help determine the adequacy of the respirator fit:

(a) Chin properly placed;

(b) Adequate strap tension, not overly tightened;

(c) Fit across nose bridge;

(d) Respirator of proper size to span distance from nose to chin;

(e) Tendency of respirator to slip;

(f) Self-observation in mirror to evaluate fit and respirator position.

8. The test subject shall conduct a user seal check, either the negative and positive pressure

seal checks described in Appendix B-1 of this section or those recommended by the respirator

manufacturer which provide equivalent protection to the procedures in Appendix B-1. Before

conducting the negative and positive pressure checks, the subject shall be told to seat the mask

on the face by moving the head from side-to-side and up and down slowly while taking in a few

slow deep breaths. Another facepiece shall be selected and retested if the test subject fails the

user seal check tests.

9. The test shall not be conducted if there is any hair growth between the skin and the facepiece

sealing surface, such as stubble beard growth, beard, mustache or sideburns which cross the

respirator sealing surface. Any type of apparel which interferes with a satisfactory fit shall be

altered or removed.

10. If a test subject exhibits difficulty in breathing during the tests, she or he shall be referred to a physician or other licensed health care professional, as appropriate, to determine whether the

test subject can wear a respirator while performing her or his duties.

11. If the employee finds the fit of the respirator unacceptable, the test subject shall be given the

opportunity to select a different respirator and to be retested.

12. Exercise regimen. Prior to the commencement of the fit test, the test subject shall be given a

description of the fit test and the test subject's responsibilities during the test procedure. The

description of the process shall include a description of the test exercises that the subject will be

performing. The respirator to be tested shall be worn for at least 5 minutes before the start of the fit test.

13. The fit test shall be performed while the test subject is wearing any applicable safety

equipment that may be worn during actual respirator use which could interfere with respirator fit.

14. Test Exercises. (a) The following test exercises are to be performed for all fit testing

methods prescribed in this appendix, except for the CNP method. A separate fit testing exercise

regimen is contained in the CNP protocol. The test subject shall perform exercises, in the test

environment, in the following manner:

(1) Normal breathing. In a normal standing position, without talking, the subject shall breathe

normally.

(2) Deep breathing. In a normal standing position, the subject shall breathe slowly and deeply,

taking caution so as not to hyperventilate.

(3) Turning head side to side. Standing in place, the subject shall slowly turn his/her head from

side to side between the extreme positions on each side. The head shall be held at each

extreme momentarily so the subject can inhale at each side.

(4) Moving head up and down. Standing in place, the subject shall slowly move his/her head up

and down. The subject shall be instructed to inhale in the up position (i.e., when looking toward

the ceiling).

(5) Talking. The subject shall talk out loud slowly and loud enough so as to be heard clearly by

the test conductor. The subject can read from a prepared text such as the Rainbow Passage,

count backward from 100, or recite a memorized poem or song.

Rainbow Passage

When the sunlight strikes raindrops in the air, they act like a prism and form a rainbow. The

rainbow is a division of white light into many beautiful colors. These take the shape of a long

round arch, with its path high above, and its two ends apparently beyond the horizon. There is,

according to legend, a boiling pot of gold at one end. People look, but no one ever finds it. When a man looks for something beyond reach, his friends say he is looking for the pot of gold at the end of the rainbow.

(6) Grimace. The test subject shall grimace by smiling or frowning. (This applies only to QNFT

testing; it is not performed for QLFT)

(7) Bending over. The test subject shall bend at the waist as if he/she were to touch his/her toes. Jogging in place shall be substituted for this exercise in those test environments such as shroud type QNFT or QLFT units that do not permit bending over at the waist.

(8) Normal breathing. Same as exercise (1).

(b) Each test exercise shall be performed for one minute except for the grimace exercise which

shall be performed for 15 seconds. The test subject shall be questioned by the test conductor

regarding the comfort of the respirator upon completion of the protocol. If it has become

unacceptable, another model of respirator shall be tried. The respirator shall not be adjusted

once the fit test exercises begin. Any adjustment voids the test, and the fit test must be

repeated.

B. Qualitative Fit Test (QLFT) Protocols

1. General

(a) The employer shall ensure that persons administering QLFT are able to prepare test

solutions, calibrate equipment and perform tests properly, recognize invalid tests, and ensure

that test equipment is in proper working order.

(b) The employer shall ensure that QLFT equipment is kept clean and well maintained so as to

operate within the parameters for which it was designed.

2. Isoamyl Acetate Protocol

Note: This protocol is not appropriate to use for the fit testing of particulate respirators. If used to

fit test particulate respirators, the respirator must be equipped with an organic vapor filter.

(a) Odor Threshold Screening

Odor threshold screening, performed without wearing a respirator, is intended to determine if the individual tested can detect the odor of isoamyl acetate at low levels.

(1) Three 1 liter glass jars with metal lids are required.

(2) Odor-free water (e.g., distilled or spring water) at approximately 25 deg. C (77 deg. F) shall

be used for the solutions.

(3) The isoamyl acetate (IAA) (also known at isopentyl acetate) stock solution is prepared by

adding 1 ml of pure IAA to 800 ml of odor-free water in a 1 liter jar, closing the lid and shaking for 30 seconds. A new solution shall be prepared at least weekly.

(4) The screening test shall be conducted in a room separate from the room used for actual fit

testing. The two rooms shall be well-ventilated to prevent the odor of IAA from becoming evident

in the general room air where testing takes place.

(5) The odor test solution is prepared in a second jar by placing 0.4 ml of the stock solution into

500 ml of odor-free water using a clean dropper or pipette. The solution shall be shaken for 30

seconds and allowed to stand for two to three minutes so that the IAA concentration above the

liquid may reach equilibrium. This solution shall be used for only one day.

(6) A test blank shall be prepared in a third jar by adding 500 cc of odor-free water.

(7) The odor test and test blank jar lids shall be labeled (e.g., 1 and 2) for jar identification.

Labels shall be placed on the lids so that they can be peeled off periodically and switched to

maintain the integrity of the test.

(8) The following instruction shall be typed on a card and placed on the table in front of the two

test jars (i.e., 1 and 2): "The purpose of this test is to determine if you can smell banana oil at a

low concentration. The two bottles in front of you contain water. One of these bottles also

contains a small amount of banana oil. Be sure the covers are on tight, then shake each bottle

for two seconds. Unscrew the lid of each bottle, one at a time, and sniff at the mouth of the

bottle. Indicate to the test conductor which bottle contains banana oil." (9) The mixtures used in

the IAA odor detection test shall be prepared in an area separate from where the test is

performed, in order to prevent olfactory fatigue in the subject. (10) If the test subject is unable to

correctly identify the jar containing the odor test solution, the IAA qualitative fit test shall not be

performed.

(11) If the test subject correctly identifies the jar containing the odor test solution, the test subject may proceed to respirator selection and fit testing.

(b) Isoamyl Acetate Fit Test

(1) The fit test chamber shall be a clear 55-gallon drum liner suspended inverted over a 2-foot

diameter frame so that the top of the chamber is about 6 inches above the test subject's head. If

no drum liner is available, a similar chamber shall be constructed using plastic sheeting. The

inside top center of the chamber shall have a small hook attached.

(2) Each respirator used for the fitting and fit testing shall be equipped with organic vapor

cartridges or offer protection against organic vapors.

(3) After selecting, donning, and properly adjusting a respirator, the test subject shall wear it to

the fit testing room. This room shall be separate from the room used for odor threshold

screening and respirator selection, and shall be well-ventilated, as by an exhaust fan or lab

hood, to prevent general room contamination.

(4) A copy of the test exercises and any prepared text from which the subject is to read shall be

taped to the inside of the test chamber.

(5) Upon entering the test chamber, the test subject shall be given a 6-inch by 5-inch piece of

paper towel, or other porous, absorbent, single-ply material, folded in half and wetted with 0.75

ml of pure IAA. The test subject shall hang the wet towel on the hook at the top of the chamber.

An IAA test swab or ampule may be substituted for the IAA wetted paper towel provided it has

been demonstrated that the alternative IAA source will generate an IAA test atmosphere with a

concentration equivalent to that generated by the paper towel method.

(6) Allow two minutes for the IAA test concentration to stabilize before starting the fit test

exercises. This would be an appropriate time to talk with the test subject; to explain the fit test,

the importance of his/her cooperation, and the purpose for the test exercises; or to demonstrate

some of the exercises.

(7) If at any time during the test, the subject detects the banana-like odor of IAA, the test is

failed. The subject shall quickly exit from the test chamber and leave the test area to avoid

olfactory fatigue.

(8) If the test is failed, the subject shall return to the selection room and remove the respirator.

The test subject shall repeat the odor sensitivity test, select and put on another respirator, return

to the test area and again begin the fit test procedure described in (b) (1) through (7) above. The process continues until a respirator that fits well has been found. Should the odor sensitivity test be failed, the subject shall wait at least 5 minutes before retesting. Odor sensitivity will usually have returned by this time.

(9) If the subject passes the test, the efficiency of the test procedure shall be demonstrated by

having the subject break the respirator face seal and take a breath before exiting the chamber.

(10) When the test subject leaves the chamber, the subject shall remove the saturated towel and return it to the person conducting the test, so that there is no significant IAA concentration

buildup in the chamber during subsequent tests. The used towels shall be kept in a self-sealing

plastic bag to keep the test area from being contaminated.

3. Saccharin Solution Aerosol Protocol

The entire screening and testing procedure shall be explained to the test subject prior to the

conduct of the screening test.

(a) Taste threshold screening. The saccharin taste threshold screening, performed without

wearing a respirator, is intended to determine whether the individual being tested can detect the

taste of saccharin.

(1) During threshold screening as well as during fit testing, subjects shall wear an enclosure

about the head and shoulders that is approximately 12 inches in diameter by 14 inches tall with

at least the front portion clear and that allows free movements of the head when a respirator is

worn. An enclosure substantially similar to the 3M hood assembly, parts # FT 14 and # FT 15

combined, is adequate.

(2) The test enclosure shall have a 3/4-inch (1.9 cm) hole in front of the test subject's nose and

mouth area to accommodate the nebulizer nozzle.

(3) The test subject shall don the test enclosure. Throughout the threshold screening test, the

test subject shall breathe through his/her slightly open mouth with tongue extended. The subject

is instructed to report when he/she detects a sweet taste.

(4) Using a DeVilbiss Model 40 Inhalation Medication Nebulizer or equivalent, the test conductor

shall spray the threshold check solution into the enclosure. The nozzle is directed away from the

nose and mouth of the person. This nebulizer shall be clearly marked to distinguish it from the fit

test solution nebulizer.

(5) The threshold check solution is prepared by dissolving 0.83 gram of sodium saccharin USP

in 100 ml of warm water. It can be prepared by putting 1 ml of the fit test solution (see (b)(5)

below) in 100 ml of distilled water.

(6) To produce the aerosol, the nebulizer bulb is firmly squeezed so that it collapses completely,

then released and allowed to fully expand. (7) Ten squeezes are repeated rapidly and then the

test subject is asked whether the saccharin can be tasted. If the test subject reports tasting the

sweet taste during the ten squeezes, the screening test is completed. The taste threshold is

noted as ten regardless of the number of squeezes actually completed.

(8) If the first response is negative, ten more squeezes are repeated rapidly and the test subject

is again asked whether the saccharin is tasted. If the test subject reports tasting the sweet taste

during the second ten squeezes, the screening test is completed. The taste threshold is noted

as twenty regardless of the number of squeezes actually completed.

(9) If the second response is negative, ten more squeezes are repeated rapidly and the test

subject is again asked whether the saccharin is tasted. If the test subject reports tasting the

sweet taste during the third set of ten squeezes, the screening test is completed. The taste

threshold is noted as thirty regardless of the number of squeezes actually completed.

(10) The test conductor will take note of the number of squeezes required to solicit a taste

response.

(11) If the saccharin is not tasted after 30 squeezes (step 10), the test subject is unable to taste

saccharin and may not perform the saccharin fit test.

Note to paragraph 3. (a): If the test subject eats or drinks something sweet before the screening

test, he/she may be unable to taste the weak saccharin solution.

(12) If a taste response is elicited, the test subject shall be asked to take note of the taste for

reference in the fit test.

(13) Correct use of the nebulizer means that approximately 1 ml of liquid is used at a time in the

nebulizer body.

(14) The nebulizer shall be thoroughly rinsed in water, shaken dry, and refilled at least each

morning and afternoon or at least every four hours.

(b) Saccharin solution aerosol fit test procedure.

(1) The test subject may not eat, drink (except plain water), smoke, or chew gum for 15 minutes

before the test.

(2) The fit test uses the same enclosure described in 3. (a) above.

(3) The test subject shall don the enclosure while wearing the respirator selected in section I. A.

of this appendix. The respirator shall be properly adjusted and equipped with a particulate

filter(s).

(4) A second DeVilbiss Model 40 Inhalation Medication Nebulizer or equivalent is used to spray

the fit test solution into the enclosure. This nebulizer shall be clearly marked to distinguish it from the screening test solution nebulizer.

(5) The fit test solution is prepared by adding 83 grams of sodium saccharin to 100 ml of warm

water.

(6) As before, the test subject shall breathe through the slightly open mouth with tongue

extended, and report if he/she tastes the sweet taste of saccharin.

(7) The nebulizer is inserted into the hole in the front of the enclosure and an initial concentration of saccharin fit test solution is sprayed into the enclosure using the same number of squeezes (either 10, 20 or 30 squeezes) based on the number of squeezes required to elicit a taste response as noted during the screening test. A minimum of 10 squeezes is required.

(8) After generating the aerosol, the test subject shall be instructed to perform the exercises in

section I. A. 14. of this appendix.

(9) Every 30 seconds the aerosol concentration shall be replenished using one half the original

number of squeezes used initially (e.g., 5, 10 or 15).

(10) The test subject shall indicate to the test conductor if at any time during the fit test the taste

of saccharin is detected. If the test subject does not report tasting the saccharin, the test is

passed.

(11) If the taste of saccharin is detected, the fit is deemed unsatisfactory and the test is failed. A

different respirator shall be tried and the entire test procedure is repeated (taste threshold

screening and fit testing).

(12) Since the nebulizer has a tendency to clog during use, the test operator must make periodic checks of the nebulizer to ensure that it is not clogged. If clogging is found at the end of the test session, the test is invalid.

4. BitrexTM (Denatonium Benzoate) Solution Aerosol Qualitative Fit Test Protocol

The BitrexTM (Denatonium benzoate) solution aerosol QLFT protocol uses the published

saccharin test protocol because that protocol is widely accepted. Bitrex is routinely used as a

taste aversion agent in household liquids which children should not be drinking and is endorsed

by the American Medical Association, the National Safety Council, and the American Association of Poison Control Centers. The entire screening and testing procedure shall be explained to the test subject prior to the conduct of the screening test.

(a) Taste Threshold Screening.

The Bitrex taste threshold screening, performed without wearing a respirator, is intended to

determine whether the individual being tested can detect the taste of Bitrex.

(1) During threshold screening as well as during fit testing, subjects shall wear an enclosure

about the head and shoulders that is approximately 12 inches (30.5 cm) in diameter by 14

inches (35.6 cm) tall. The front portion of the enclosure shall be clear from the respirator and

allow free movement of the head when a respirator is worn. An enclosure substantially similar to

the 3M hood assembly, parts # FT 14 and # FT 15 combined, is adequate.

(2) The test enclosure shall have a \3/4\ inch (1.9 cm) hole in front of the test subject's nose and

mouth area to accommodate the nebulizer nozzle.

(3) The test subject shall don the test enclosure. Throughout the threshold screening test, the test subject shall breathe through his or her slightly open mouth with tongue extended. The subject is instructed to report when he/she detects a bitter taste

(4) Using a DeVilbiss Model 40 Inhalation Medication Nebulizer or equivalent, the test conductor

shall spray the Threshold Check Solution into the enclosure. This Nebulizer shall be clearly

marked to distinguish it from the fit test solution nebulizer.

(5) The Threshold Check Solution is prepared by adding 13.5 milligrams of Bitrex to 100 ml of

5% salt (NaCl) solution in distilled water.

(6) To produce the aerosol, the nebulizer bulb is firmly squeezed so that the bulb collapses

completely, and is then released and allowed to fully expand.

(7) An initial ten squeezes are repeated rapidly and then the test subject is asked whether the

Bitrex can be tasted. If the test subject reports tasting the bitter taste during the ten squeezes,

the screening test is completed. The taste threshold is noted as ten regardless of the number of

squeezes actually completed.

(8) If the first response is negative, ten more squeezes are repeated rapidly and the test subject

is again asked whether the Bitrex is tasted. If the test subject reports tasting the bitter taste

during the second ten squeezes, the screening test is completed. The taste threshold is noted

as twenty regardless of the number of squeezes actually completed.

(9) If the second response is negative, ten more squeezes are repeated rapidly and the test

subject is again asked whether the Bitrex is tasted. If the test subject reports tasting the bitter

taste during the third set of ten squeezes, the screening test is completed. The taste threshold is

noted as thirty regardless of the number of squeezes actually completed.

(10) The test conductor will take note of the number of squeezes required to solicit a taste

response.

(11) If the Bitrex is not tasted after 30 squeezes (step 10), the test subject is unable to taste

Bitrex and may not perform the Bitrex fit test.

(12) If a taste response is elicited, the test subject shall be asked to take note of the taste for

reference in the fit test.

(13) Correct use of the nebulizer means that approximately 1 ml of liquid is used at a time in the

nebulizer body.

(14) The nebulizer shall be thoroughly rinsed in water, shaken to dry, and refilled at least each

morning and afternoon or at least every four hours.

(b) Bitrex Solution Aerosol Fit Test Procedure.

(1) The test subject may not eat, drink (except plain water), smoke, or chew gum for 15 minutes

before the test.

(2) The fit test uses the same enclosure as that described in 4. (a) above.

(3) The test subject shall don the enclosure while wearing the respirator selected according to

section I. A. of this appendix. The respirator shall be properly adjusted and equipped with any

type particulate filter(s).

(4) A second DeVilbiss Model 40 Inhalation Medication Nebulizer or equivalent is used to spray

the fit test solution into the enclosure. This nebulizer shall be clearly marked to distinguish it from

the screening test solution nebulizer.

(5) The fit test solution is prepared by adding 337.5 mg of Bitrex to 200 ml of a 5% salt (NaCl)

solution in warm water.

(6) As before, the test subject shall breathe through his or her slightly open mouth with tongue

extended, and be instructed to report if he/she tastes the bitter taste of Bitrex.

(7) The nebulizer is inserted into the hole in the front of the enclosure and an initial concentration of the fit test solution is sprayed into the enclosure using the same number of squeezes (either 10, 20 or 30 squeezes) based on the number of squeezes required to elicit a taste response as noted during the screening test.

(8) After generating the aerosol, the test subject shall be instructed to perform the exercises in

section I. A. 14. of this appendix.

(9) Every 30 seconds the aerosol concentration shall be replenished using one half the number

of squeezes used initially (e.g., 5, 10 or 15).

(10) The test subject shall indicate to the test conductor if at any time during the fit test the taste

of Bitrex is detected. If the test subject does not report tasting the Bitrex, the test is passed.

(11) If the taste of Bitrex is detected, the fit is deemed unsatisfactory and the test is failed. A

different respirator shall be tried and the entire test procedure is repeated (taste threshold

screening and fit testing).

5. Irritant Smoke (Stannic Chloride) Protocol

This qualitative fit test uses a person's response to the irritating chemicals released in the

"smoke" produced by a stannic chloride ventilation smoke tube to detect leakage into the

respirator.

(a) General Requirements and Precautions

(1) The respirator to be tested shall be equipped with high efficiency particulate air (HEPA) or

P100 series filter(s).

(2) Only stannic chloride smoke tubes shall be used for this protocol.

(3) No form of test enclosure or hood for the test subject shall be used.

(4) The smoke can be irritating to the eyes, lungs, and nasal passages. The test conductor shall

take precautions to minimize the test subject's exposure to irritant smoke. Sensitivity varies, and

certain individuals may respond to a greater degree to irritant smoke. Care shall be taken when

performing the sensitivity screening checks that determine whether the test subject can detect

irritant smoke to use only the minimum amount of smoke necessary to elicit a response from the

test subject.

(5) The fit test shall be performed in an area with adequate ventilation to prevent exposure of the person conducting the fit test or the build-up of irritant smoke in the general atmosphere.

(b) Sensitivity Screening Check

The person to be tested must demonstrate his or her ability to detect a weak concentration of the irritant smoke.

(1) The test operator shall break both ends of a ventilation smoke tube containing stannic

chloride, and attach one end of the smoke tube to a low flow air pump set to deliver 200

milliliters per minute, or an aspirator squeeze bulb. The test operator shall cover the other end of the smoke tube with a short piece of tubing to prevent potential injury from the jagged end of the smoke tube.

(2) The test operator shall advise the test subject that the smoke can be irritating to the eyes,

lungs, and nasal passages and instruct the subject to keep his/her eyes closed while the test is

performed.

(3) The test subject shall be allowed to smell a weak concentration of the irritant smoke before

the respirator is donned to become familiar with its irritating properties and to determine if he/she can detect the irritating properties of the smoke. The test operator shall carefully direct a small amount of the irritant smoke in the test subject's direction to determine that he/she can detect it.

(c) Irritant Smoke Fit Test Procedure

(1) The person being fit tested shall don the respirator without assistance, and perform the

required user seal check(s).

(2) The test subject shall be instructed to keep his/her eyes closed.

(3) The test operator shall direct the stream of irritant smoke from the smoke tube toward the

faceseal area of the test subject, using the low flow pump or the squeeze bulb. The test operator shall begin at least 12 inches from the facepiece and move the smoke stream around the whole perimeter of the mask. The operator shall gradually make two more passes around the perimeter of the mask, moving to within six inches of the respirator.

(4) If the person being tested has not had an involuntary response and/or detected the irritant

smoke, proceed with the test exercises.

(5) The exercises identified in section I.A. 14. of this appendix shall be performed by the test

subject while the respirator seal is being continually challenged by the smoke, directed around

the perimeter of the respirator at a distance of six inches.

(6) If the person being fit tested reports detecting the irritant smoke at any time, the test is failed.

The person being retested must repeat the entire sensitivity check and fit test procedure.

(7) Each test subject passing the irritant smoke test without evidence of a response (involuntary

cough, irritation) shall be given a second sensitivity screening check, with the smoke from the

same smoke tube used during the fit test, once the respirator has been removed, to determine

whether he/she still reacts to the smoke. Failure to evoke a response shall void the fit test.

(8) If a response is produced during this second sensitivity check, then the fit test is passed.

C. Quantitative Fit Test (QNFT) Protocols

The following quantitative fit testing procedures have been demonstrated to be acceptable:

Quantitative fit testing using a non-hazardous test aerosol (such as corn oil, polyethylene glycol

400 [PEG 400], di-2-ethyl hexyl sebacate [DEHS], or sodium chloride) generated in a test

chamber, and employing instrumentation to quantify the fit of the respirator; Quantitative fit

testing using ambient aerosol as the test agent and appropriate instrumentation (condensation

nuclei counter) to quantify the respirator fit; Quantitative fit testing using controlled negative

pressure and appropriate instrumentation to measure the volumetric leak rate of a facepiece to

quantify the respirator fit.

1. General

(a) The employer shall ensure that persons administering QNFT are able to calibrate equipment

and perform tests properly, recognize invalid tests, calculate fit factors properly and ensure that

test equipment is in proper working order.

(b) The employer shall ensure that QNFT equipment is kept clean, and is maintained and

calibrated according to the manufacturer's instructions so as to operate at the parameters for

which it was designed.

2. Generated Aerosol Quantitative Fit Testing Protocol

(a) Apparatus.

(1) Instrumentation. Aerosol generation, dilution, and measurement systems using particulates

(corn oil, polyethylene glycol 400 [PEG 400], di-2-ethyl hexyl sebacate [DEHS] or sodium

chloride) as test aerosols shall be used for quantitative fit testing.

(2) Test chamber. The test chamber shall be large enough to permit all test subjects to perform

freely all required exercises without disturbing the test agent concentration or the measurement

apparatus. The test chamber shall be equipped and constructed so that the test agent is

effectively isolated from the ambient air, yet uniform in concentration throughout the chamber.

(3) When testing air-purifying respirators, the normal filter or cartridge element shall be replaced

with a high efficiency particulate air (HEPA) or P100 series filter supplied by the same

manufacturer.

(4) The sampling instrument shall be selected so that a computer record or strip chart record

may be made of the test showing the rise and fall of the test agent concentration with each

inspiration and expiration at fit factors of at least 2,000. Integrators or computers that integrate

the amount of test agent penetration leakage into the respirator for each exercise may be used

provided a record of the readings is made.

(5) The combination of substitute air-purifying elements, test agent and test agent concentration

shall be such that the test subject is not exposed in excess of an established exposure limit for

the test agent at any time during the testing process, based upon the length of the exposure and

the exposure limit duration.

(6) The sampling port on the test specimen respirator shall be placed and constructed so that no

leakage occurs around the port (e.g., where the respirator is probed), a free air flow is allowed

into the sampling line at all times, and there is no interference with the fit or performance of the

respirator. The in-mask sampling device (probe) shall be designed and used so that the air

sample is drawn from the breathing zone of the test subject, midway between the nose and

mouth and with the probe extending into the facepiece cavity at least 1/4 inch.

(7) The test setup shall permit the person administering the test to observe the test subject

inside the chamber during the test.

(8) The equipment generating the test atmosphere shall maintain the concentration of test agent

constant to within a 10 percent variation for the duration of the test.

(9) The time lag (interval between an event and the recording of the event on the strip chart or

computer or integrator) shall be kept to a minimum. There shall be a clear association between

the occurrence of an event and its being recorded.

(10) The sampling line tubing for the test chamber atmosphere and for the respirator sampling

port shall be of equal diameter and of the same material. The length of the two lines shall be

equal.

(11) The exhaust flow from the test chamber shall pass through an appropriate filter (i.e., high

efficiency particulate filter) before release.

(12) When sodium chloride aerosol is used, the relative humidity inside the test chamber shall

not exceed 50 percent.

(13) The limitations of instrument detection shall be taken into account when determining the fit

factor.

(14) Test respirators shall be maintained in proper working order and be inspected regularly for

deficiencies such as cracks or missing valves and gaskets.

(b) Procedural Requirements.

(1) When performing the initial user seal check using a positive or negative pressure check, the

sampling line shall be crimped closed in order to avoid air pressure leakage during either of

these pressure checks.

(2) The use of an abbreviated screening QLFT test is optional. Such a test may be utilized in

order to quickly identify poor fitting respirators that passed the positive and/or negative pressure

test and reduce the amount of QNFT time. The use of the CNC QNFT instrument in the count

mode is another optional method to obtain a quick estimate of fit and eliminate poor fitting

respirators before going on to perform a full QNFT.

(3) A reasonably stable test agent concentration shall be measured in the test chamber prior to

testing. For canopy or shower curtain types of test units, the determination of the test agent's

stability may be established after the test subject has entered the test environment.

(4) Immediately after the subject enters the test chamber, the test agent concentration inside the

respirator shall be measured to ensure that the peak penetration does not exceed 5 percent for

a half mask or 1 percent for a full facepiece respirator.

(5) A stable test agent concentration shall be obtained prior to the actual start of testing.

(6) Respirator restraining straps shall not be over-tightened for testing. The straps shall be

adjusted by the wearer without assistance from other persons to give a reasonably comfortable

fit typical of normal use. The respirator shall not be adjusted once the fit test exercises begin.

(7) The test shall be terminated whenever any single peak penetration exceeds 5 percent for half masks and 1 percent for full facepiece respirators. The test subject shall be refitted and retested.

(8) Calculation of fit factors.

(i) The fit factor shall be determined for the quantitative fit test by taking the ratio of the average

chamber concentration to the concentration measured inside the respirator for each test

exercise

except the grimace exercise.

(ii) The average test chamber concentration shall be calculated as the arithmetic average of the

concentration measured before and after each test (i.e., 7 exercises) or the arithmetic average

of the concentration measured before and after each exercise or the true average measured

continuously during the respirator sample.

(iii) The concentration of the challenge agent inside the respirator shall be determined by one of

the following methods:

(A) Average peak penetration method means the method of determining test agent penetration

into the respirator utilizing a strip chart recorder, integrator, or computer. The agent penetration

is determined by an average of the peak heights on the graph or by computer integration, for

each exercise except the grimace exercise. Integrators or computers that calculate the actual

test agent penetration into the respirator for each exercise will also be considered to meet the

requirements of the average peak penetration method.

(B) Maximum peak penetration method means the method of determining test agent penetration

in the respirator as determined by strip chart recordings of the test. The highest peak penetration for a given exercise is taken to be representative of average penetration into the respirator for that exercise.

(C) Integration by calculation of the area under the individual peak for each exercise except the

grimace exercise. This includes computerized integration.

(D) The calculation of the overall fit factor using individual exercise fit factors involves first

converting the exercise fit factors to penetration values, determining the average, and then

converting that result back to a fit factor. This procedure is described in the following equation:

Overall fit factor = Number of exercises

1/ff1 +1/ff2 + 1/ff3 + 1/ff4 + 1/ff5 + 1/ff6 + 1/ff7 + 1/ff8

Where ff1, ff2, ff3, etc. are the fit factors for exercises 1, 2, 3, etc.

(9) The test subject shall not be permitted to wear a half mask or quarter facepiece respirator

unless a minimum fit factor of 100 is obtained, or a full facepiece respirator unless a minimum fit

factor of 500 is obtained.

(10) Filters used for quantitative fit testing shall be replaced whenever increased breathing

resistance is encountered, or when the test agent has altered the integrity of the filter media.

3. Ambient aerosol condensation nuclei counter (CNC) quantitative fit testing protocol.

The ambient aerosol condensation nuclei counter (CNC) quantitative fit testing (Portacount TM )

protocol quantitatively fit tests respirators with the use of a probe. The probed respirator is only

used for quantitative fit tests. A probed respirator has a special sampling device, installed on the

respirator, that allows the probe to sample the air from inside the mask. A probed respirator is

required for each make, style, model, and size that the employer uses and can be obtained from

the respirator manufacturer or distributor. The CNC instrument manufacturer, TSI Inc., also

provides probe attachments (TSI sampling adapters) that permit fit testing in an employee's own

respirator. A minimum fit factor pass level of at least 100 is necessary for a half-mask respirator

and a minimum fit factor pass level of at least 500 is required for a full facepiece negative

pressure respirator. The entire screening and testing procedure shall be explained to the test

subject prior to the conduct of the screening test. (a) Portacount Fit Test Requirements.

(1) Check the respirator to make sure the sampling probe and line are properly attached to the

facepiece and that the respirator is fitted with a particulate filter capable of preventing significant

penetration by the ambient particles used for the fit test (e.g., NIOSH 42 CFR 84 series 100,

series 99, or series 95 particulate filter) per manufacturer's instruction.

(2) Instruct the person to be tested to don the respirator for five minutes before the fit test starts.

This purges the ambient particles trapped inside the respirator and permits the wearer to make

certain the respirator is comfortable. This individual shall already have been trained on how to

wear the respirator properly.

(3) Check the following conditions for the adequacy of the respirator fit: Chin properly placed;

Adequate strap tension, not overly tightened; Fit across nose bridge; Respirator of proper size to span distance from nose to chin; Tendency of the respirator to slip; Self-observation in a mirror to evaluate fit and respirator position.

(4) Have the person wearing the respirator do a user seal check. If leakage is detected,

determine the cause. If leakage is from a poorly fitting facepiece, try another size of the same

model respirator, or another model of respirator.

(5) Follow the manufacturer's instructions for operating the Portacount and proceed with the test.

(6) The test subject shall be instructed to perform the exercises in section I. A. 14. of this

appendix.

(7) After the test exercises, the test subject shall be questioned by the test conductor regarding

the comfort of the respirator upon completion of the protocol. If it has become unacceptable,

another model of respirator shall be tried.

(b) Portacount Test Instrument.

(1) The Portacount will automatically stop and calculate the overall fit factor for the entire set of

exercises. The overall fit factor is what counts. The Pass or Fail message will indicate whether or not the test was successful. If the test was a Pass, the fit test is over.

(2) Since the pass or fail criterion of the Portacount is user programmable, the test operator shall ensure that the pass or fail criterion meet the requirements for minimum respirator performance in this Appendix.

(3) A record of the test needs to be kept on file, assuming the fit test was successful. The record

must contain the test subject's name; overall fit factor; make, model, style, and size of respirator

used; and date tested.

4. Controlled negative pressure (CNP) quantitative fit testing protocol. The CNP protocol

provides an alternative to aerosol fit test methods. The CNP fit test method technology is based

on exhausting air from a temporarily sealed respirator facepiece to generate and then maintain a constant negative pressure inside the facepiece. The rate of air exhaust is controlled so that a

constant negative pressure is maintained in the respirator during the fit test. The level of

pressure is selected to replicate the mean inspiratory pressure that causes leakage into the

respirator under normal use conditions. With pressure held constant, air flow out of the respirator is equal to air flow into the respirator. Therefore, measurement of the exhaust stream that is required to hold the pressure in the temporarily sealed respirator constant yields a direct measure of leakage air flow into the respirator. The CNP fit test method measures leak rates through the facepiece as a method for determining the facepiece fit for negative pressure respirators. The CNP instrument manufacturer Dynatech Nevada also provides attachments (sampling manifolds) that replace the filter cartridges to permit fit testing in an employee's own respirator. To perform the test, the test subject closes his or her mouth and holds his/her breath, after which an air pump removes air from the respirator facepiece at a pre-selected constant pressure. The facepiece fit is expressed as the leak rate through the facepiece, expressed as milliliters per minute. The quality and validity of the CNP fit tests are determined by the degree to which the in-mask pressure tracks the test pressure during the system measurement time of approximately five seconds. Instantaneous feedback in the form of a real-time pressure trace of the in-mask pressure is provided and used to determine test validity and quality. A minimum fit factor pass level of 100 is necessary for a half-mask respirator and a minimum fit factor of at least 500 is required for a full facepiece respirator. The entire screening and testing procedure shall be explained to the test subject prior to the conduct of the screening test.

(a) CNP Fit Test Requirements.

(1) The instrument shall have a non-adjustable test pressure of 15.0 mm water pressure.

(2) The CNP system defaults selected for test pressure shall be set at -- 15 mm of water (-0.58

inches of water) and the modeled inspiratory flow rate shall be 53.8 liters per minute for

performing fit tests.

(Note: CNP systems have built-in capability to conduct fit testing that is specific to unique work

rate, mask, and gender situations that might apply in a specific workplace. Use of system default values, which were selected to represent respirator wear with medium cartridge resistance at a low-moderate work rate, will allow inter-test comparison of the respirator fit.)

(3) The individual who conducts the CNP fit testing shall be thoroughly trained to perform the

test.

(4) The respirator filter or cartridge needs to be replaced with the CNP test manifold. The

inhalation valve downstream from the manifold either needs to be temporarily removed or

propped open.

(5) The test subject shall be trained to hold his or her breath for at least 20

seconds.

(6) The test subject shall don the test respirator without any assistance from the individual who

conducts the CNP fit test.

(7) The QNFT protocol shall be followed according to section I.C. 1. of this appendix with an

exception for the CNP test exercises.

(b) CNP Test Exercises.

(1) Normal breathing. In a normal standing position, without talking, the subject shall breathe

normally for 1 minute. After the normal breathing exercise, the subject needs to hold head

straight ahead and hold his or her breath for 10 seconds during the test measurement.

(2) Deep breathing. In a normal standing position, the subject shall breathe slowly and deeply for 1 minute, being careful not to hyperventilate. After the deep breathing exercise, the subject shall hold his or her head straight ahead and hold his or her breath for 10 seconds during test

measurement.

(3) Turning head side to side. Standing in place, the subject shall slowly turn his or her head

from side to side between the extreme positions on each side for 1 minute. The head shall be

held at each extreme momentarily so the subject can inhale at each side. After the turning head

side to side exercise, the subject needs to hold head full left and hold his or her breath for 10

seconds during test measurement. Next, the subject needs to hold head full right and hold his or

her breath for 10 seconds during test measurement.

(4) Moving head up and down. Standing in place, the subject shall slowly move his or her head

up and down for 1 minute. The subject shall be instructed to inhale in the up position (i.e., when

looking toward the ceiling). After the moving head up and down exercise, the subject shall hold

his or her head full up and hold his or her breath for 10 seconds during test measurement. Next,

the subject shall hold his or her head full down and hold his or her breath for 10 seconds during

test measurement.

(5) Talking. The subject shall talk out loud slowly and loud enough so as to be heard clearly by

the test conductor. The subject can read from a prepared text such as the Rainbow Passage,

count backward from 100, or

recite a memorized poem or song for 1 minute. After the talking

exercise, the subject shall hold his or her head straight ahead and hold his or her breath for 10

seconds during the test measurement.

(6) Grimace. The test subject shall grimace by smiling or frowning for 15 seconds.

(7) Bending Over. The test subject shall bend at the waist as if he or she were to touch his or her toes for 1 minute. Jogging in place shall be substituted for this exercise in those test

environments such as shroud-type QNFT units that prohibit bending at the waist. After the

bending over exercise, the subject shall hold his or her head straight ahead and hold his or her

breath for 10 seconds during the test measurement.

(8) Normal Breathing. The test subject shall remove and re-don the respirator within a

one-minute period. Then, in a normal standing position, without talking, the subject shall breathe

normally for 1 minute. After the normal breathing exercise, the subject shall hold his or her head

straight ahead and hold his or her breath for 10 seconds during the test measurement. After the

test exercises, the test subject shall be questioned by the test conductor regarding the comfort of the respirator upon completion of the protocol. If it has become unacceptable, another model of a respirator shall be tried.

(c) CNP Test Instrument.

(1) The test instrument shall have an effective audio warning device when the test subject fails to hold his or her breath during the test. The test shall be terminated whenever the test subject

failed to hold his or her breath. The test subject may be refitted and retested. (2) A record of the

test shall be kept on file, assuming the fit test was successful. The record must contain the test

subject's name; overall fit factor; make, model, style and size of respirator used; and date tested.

Part II. New Fit Test Protocols

A. Any person may submit to OSHA an application for approval of a new fit test protocol. If the

application meets the following criteria, OSHA will initiate a rulemaking proceeding under section 6(b)(7) of the OSH Act to determine whether to list the new protocol as an approved protocol in this Appendix A. B. The application must include a detailed description of the proposed new fit test protocol. This application must be supported by either:

1. A test report prepared by an independent government research laboratory (e.g., Lawrence

Livermore National Laboratory, Los Alamos National Laboratory, the National Institute for

Standards and Technology) stating that the laboratory has tested the protocol and had found it

to be accurate and reliable; or

2. An article that has been published in a peer-reviewed industrial hygiene journal describing the

protocol and explaining how test data support the protocol's accuracy and reliability.

3. If OSHA determines that additional information is required before the Agency commences a

rulemaking proceeding under this section, OSHA will so notify the applicant and afford the

applicant the opportunity to submit the supplemental information. Initiation of a rulemaking

proceeding will be deferred until OSHA has received and evaluated the supplemental

information.

Appendix B-1: User Seal Check Procedures (Mandatory)

The individual who uses a tight-fitting respirator is to perform a user seal check to ensure that an adequate seal is achieved each time the respirator is put on. Either the positive and negative

pressure checks listed in this appendix, or the respirator manufacturer's recommended user seal check method shall be used. User seal checks are not substitutes for qualitative or quantitative fit tests.

I. Facepiece Positive and/or Negative Pressure Checks

A. Positive pressure check. Close off the exhalation valve and exhale gently into the facepiece.

The face fit is considered satisfactory if a slight positive pressure can be built up inside the

facepiece without any evidence of outward leakage of air at the seal. For most respirators this

method of leak testing requires the wearer to first remove the exhalation valve cover before

closing off the exhalation valve and then carefully replacing it after the test.

B. Negative pressure check. Close off the inlet opening of the canister or cartridge(s) by

covering with the palm of the hand(s) or by replacing the filter seal(s), inhale gently so that the

facepiece collapses slightly, and hold the breath for ten seconds. The design of the inlet opening of some cartridges cannot be effectively covered with the palm of the hand. The test can be performed by covering the inlet opening of the cartridge with a thin latex or nitrile glove. If the facepiece remains in its slightly collapsed condition and no inward leakage of air is detected, the tightness of the respirator is considered satisfactory.

II. Manufacturer's Recommended User Seal Check Procedures

The respirator manufacturer's recommended procedures for performing a user seal check may

be used instead of the positive and/or negative pressure check procedures provided that the

employer demonstrates that the manufacturer's procedures are equally effective.

Appendix B-2: Respirator Cleaning Procedures (Mandatory)

These procedures are provided for employer use when cleaning respirators. They are general in

nature, and the employer as an alternative may use the cleaning recommendations provided by

the manufacturer of the respirators used by their employees, provided such procedures are as

effective as those listed here in Appendix B- 2. Equivalent effectiveness simply means that the

procedures used must accomplish the objectives set forth in Appendix B-2, i.e., must ensure that the respirator is properly cleaned and disinfected in a manner that prevents damage to the

respirator and does not cause harm to the user.

I. Procedures for Cleaning Respirators

A. Remove filters, cartridges, or canisters. Disassemble facepieces by removing speaking

diaphragms, demand and pressure- demand valve assemblies, hoses, or any components

recommended by the manufacturer. Discard or repair any defective parts.

B. Wash components in warm (43 deg. C [110 deg. F] maximum) water with a mild detergent or

with a cleaner recommended by the manufacturer. A stiff bristle (not wire) brush may be used to

facilitate the removal of dirt.

C. Rinse components thoroughly in clean, warm (43 deg. C [110 deg. F] maximum), preferably

running water. Drain.

D. When the cleaner used does not contain a disinfecting agent, respirator components should

be immersed for two minutes in one of the following:

1. Hypochlorite solution (50 ppm of chlorine) made by adding approximately one milliliter of

laundry bleach to one liter of water at 43 deg. C (110 deg. F); or,

2. Aqueous solution of iodine (50 ppm iodine) made by adding approximately 0.8 milliliters of

tincture of iodine (6-8 grams ammonium and/or potassium iodide/100 cc of 45% alcohol) to one

liter of water at 43 deg. C (110 deg. F); or,

3. Other commercially available cleansers of equivalent disinfectant quality when used as

directed, if their use is recommended or approved by the respirator manufacturer.

E. Rinse components thoroughly in clean, warm (43 deg. C [110 deg. F] maximum), preferably

running water. Drain. The importance of thorough rinsing cannot be overemphasized.

Detergents or disinfectants that dry on facepieces may result in dermatitis. In addition, some

disinfectants may cause deterioration of rubber or corrosion of metal parts if not completely

removed.

F. Components should be hand-dried with a clean lint-free cloth or air-dried.

G. Reassemble facepiece, replacing filters, cartridges, and canisters where necessary.

H. Test the respirator to ensure that all components work properly.

Appendix C: Respirator Medical Evaluation Questionnaire (Mandatory)

To the employer: Answers to questions in Section 1, and to question 9 in Section 2 of Part A, do

not require a medical examination.

To the employee:

Can you read (circle one): Yes/No

Your employer must allow you to answer this questionnaire during normal working hours, or at a

time and place that is convenient to you. To maintain your confidentiality, your employer or

supervisor must not look at or review your answers, and your employer must tell you how to

deliver or send this questionnaire to the health care professional who will review it.

Part A. Section 1. (Mandatory) The following information must be provided by every employee

who has been selected to use any type of respirator (please print).

1. Today's date:_______________________________________________________

2. Your name:__________________________________________________________

3. Your age (to nearest year):_________________________________________

4. Sex (circle one): Male/Female

5. Your height: __________ ft. __________ in.

6. Your weight: ____________ lbs.

7. Your job title:_____________________________________________________

8. A phone number where you can be reached by the health care professional who reviews this

questionnaire (include the Area Code):____________________

9. The best time to phone you at this number: ________________

10. Has your employer told you how to contact the health care professional who will review this

questionnaire (circle one): Yes/No

11. Check the type of respirator you will use (you can check more than one category):

a. ______ N, R, or P disposable respirator (filter-mask, non- cartridge type only).

b. ______ Other type (for example, half- or full-facepiece type, powered-air purifying,

supplied-air, self-contained breathing apparatus).

12. Have you worn a respirator (circle one): Yes/No

If "yes," what type(s):____________________________________________

_____________________________________________________________________

Part A. Section 2. (Mandatory) Questions 1 through 9 below must be answered by every

employee who has been selected to use any type of respirator (please circle "yes" or "no").

1. Do you currently smoke tobacco, or have you smoked tobacco in the last month: Yes/No

2. Have you ever had any of the following conditions?

a. Seizures (fits): Yes/No

b. Diabetes (sugar disease): Yes/No

c. Allergic reactions that interfere with your breathing: Yes/No

d. Claustrophobia (fear of closed-in places): Yes/No

e. Trouble smelling odors: Yes/No

3. Have you ever had any of the following pulmonary or lung problems?

a. Asbestosis: Yes/No

b. Asthma: Yes/No

c. Chronic bronchitis: Yes/No

d. Emphysema: Yes/No

e. Pneumonia: Yes/No

f. Tuberculosis: Yes/No

g. Silicosis: Yes/No

h. Pneumothorax (collapsed lung): Yes/No

i. Lung cancer: Yes/No

j. Broken ribs: Yes/No

k. Any chest injuries or surgeries: Yes/No

l. Any other lung problem that you've been told about: Yes/No

4. Do you currently have any of the following symptoms of pulmonary or lung illness?

a. Shortness of breath: Yes/No

b. Shortness of breath when walking fast on level ground or walking up a slight hill or incline:

Yes/No

c. Shortness of breath when walking with other people at an ordinary pace on level ground:

Yes/No

d. Have to stop for breath when walking at your own pace on level ground: Yes/No

e. Shortness of breath when washing or dressing yourself: Yes/No

f. Shortness of breath that interferes with your job: Yes/No

g. Coughing that produces phlegm (thick sputum): Yes/No

h. Coughing that wakes you early in the morning: Yes/No

i. Coughing that occurs mostly when you are lying down: Yes/No

j. Coughing up blood in the last month: Yes/No

k. Wheezing: Yes/No

l. Wheezing that interferes with your job: Yes/No

m. Chest pain when you breathe deeply: Yes/No

n. Any other symptoms that you think may be related to lung problems:

Yes/No

5. Have you ever had any of the following cardiovascular or heart problems?

a. Heart attack: Yes/No

b. Stroke: Yes/No

c. Angina: Yes/No

d. Heart failure: Yes/No

e. Swelling in your legs or feet (not caused by walking): Yes/No

f. Heart arrhythmia (heart beating irregularly): Yes/No

g. High blood pressure: Yes/No

h. Any other heart problem that you've been told about: Yes/No

6. Have you ever had any of the following cardiovascular or heart

symptoms?

a. Frequent pain or tightness in your chest: Yes/No

b. Pain or tightness in your chest during physical activity: Yes/No

c. Pain or tightness in your chest that interferes with your job: Yes/No

d. In the past two years, have you noticed your heart skipping or missing a beat: Yes/No

e. Heartburn or indigestion that is not related to eating: Yes/ No

f. Any other symptoms that you think may be related to heart or circulation problems: Yes/No

7. Do you currently take medication for any of the following problems?

a. Breathing or lung problems: Yes/No

b. Heart trouble: Yes/No

c. Blood pressure: Yes/No

d. Seizures (fits): Yes/No

8. If you've used a respirator, have you ever had any of the following problems? (If you've never

used a respirator, check the following space and go to question 9:)

a. Eye irritation: Yes/No

b. Skin allergies or rashes: Yes/No

c. Anxiety: Yes/No

d. General weakness or fatigue: Yes/No

e. Any other problem that interferes with your use of a respirator: Yes/No

9. Would you like to talk to the health care professional who will review this questionnaire about

your answers to this questionnaire: Yes/No

Questions 10 to 15 below must be answered by every employee who has been selected to

use either a full-facepiece respirator or a self-contained breathing apparatus (SCBA). For

employees who have been selected to use other types of respirators, answering these

questions is voluntary.

10. Have you ever lost vision in either eye (temporarily or permanently): Yes/No

11. Do you currently have any of the following vision problems?

a. Wear contact lenses: Yes/No

b. Wear glasses: Yes/No

c. Color blind: Yes/No

d. Any other eye or vision problem: Yes/No

12. Have you ever had an injury to your ears, including a broken ear drum: Yes/No

13. Do you currently have any of the following hearing problems?

a. Difficulty hearing: Yes/No

b. Wear a hearing aid: Yes/No

c. Any other hearing or ear problem: Yes/No

14. Have you ever had a back injury: Yes/No

15. Do you currently have any of the following musculoskeletal problems?

a. Weakness in any of your arms, hands, legs, or feet: Yes/No

b. Back pain: Yes/No

c. Difficulty fully moving your arms and legs: Yes/No

d. Pain or stiffness when you lean forward or backward at the waist: Yes/No

e. Difficulty fully moving your head up or down: Yes/No

f. Difficulty fully moving your head side to side: Yes/No

g. Difficulty bending at your knees: Yes/No

h. Difficulty squatting to the ground: Yes/No

i. Climbing a flight of stairs or a ladder carrying more than 25 lbs: Yes/No

j. Any other muscle or skeletal problem that interferes with using a respirator: Yes/No

Part B Any of the following questions, and other questions not listed, may be added to the

questionnaire at the discretion of the health care professional who will review the questionnaire.

1. In your present job, are you working at high altitudes (over 5,000 feet) or in a place that has

lower than normal amounts of oxygen: Yes/No

If "yes," do you have feelings of dizziness, shortness of breath,

pounding in your chest, or other symptoms when you're working under these conditions: Yes/No

2. At work or at home, have you ever been exposed to hazardous solvents, hazardous airborne

chemicals (e.g., gases, fumes, or dust), or have you come into skin contact with hazardous

chemicals: Yes/No

If "yes," name the chemicals if you know them:_________________________

_______________________________________________________________________

_______________________________________________________________________

3. Have you ever worked with any of the materials, or under any of the conditions, listed below:

a. Asbestos: Yes/No

b. Silica (e.g., in sandblasting): Yes/No

c. Tungsten/cobalt (e.g., grinding or welding this material): Yes/No

d. Beryllium: Yes/No

e. Aluminum: Yes/No

f. Coal (for example, mining): Yes/No

g. Iron: Yes/No

h. Tin: Yes/No

i. Dusty environments: Yes/No

j. Any other hazardous exposures: Yes/No

If "yes," describe these exposures:____________________________________

_______________________________________________________________________

_______________________________________________________________________

4. List any second jobs or side businesses you have:___________________

_______________________________________________________________________

5. List your previous occupations:_____________________________________

_______________________________________________________________________

6. List your current and previous hobbies:________________________________

_______________________________________________________________________

7. Have you been in the military services? Yes/No

If "yes," were you exposed to biological or chemical agents (either in training or combat): Yes/No

8. Have you ever worked on a HAZMAT team? Yes/No

9. Other than medications for breathing and lung problems, heart trouble, blood pressure, and

seizures mentioned earlier in this questionnaire, are you taking any other medications for any

reason (including over-the-counter medications): Yes/No

If "yes," name the medications if you know them:_______________________

10. Will you be using any of the following items with your respirator(s)?

a. HEPA Filters: Yes/No

b. Canisters (for example, gas masks): Yes/No

c. Cartridges: Yes/No

11. How often are you expected to use the respirator(s) (circle "yes" or "no" for all answers that

apply to you)?:

a. Escape only (no rescue): Yes/No

b. Emergency rescue only: Yes/No

c. Less than 5 hours per week: Yes/No

d. Less than 2 hours per day: Yes/No

e. 2 to 4 hours per day: Yes/No

f. Over 4 hours per day: Yes/No

12. During the period you are using the respirator(s), is your work effort:

a. Light (less than 200 kcal per hour): Yes/No

If "yes," how long does this period last during the average

shift:____________hrs.____________mins.

Examples of a light work effort are sitting while writing, typing, drafting, or performing light

assembly work; or standing while operating a drill press (1-3 lbs.) or controlling machines.

b. Moderate (200 to 350 kcal per hour): Yes/No

If "yes," how long does this period last during the average

shift:____________hrs.____________mins.

Examples of moderate work effort are sitting while nailing or filing; driving a truck or bus in urban

traffic; standing while drilling, nailing, performing assembly work, or transferring a moderate load

(about 35 lbs.) at trunk level; walking on a level surface about 2 mph or down a 5-degree grade

about 3 mph; or pushing a wheelbarrow with a heavy load (about 100 lbs.) on a level surface.

c. Heavy (above 350 kcal per hour): Yes/No

If "yes," how long does this period last during the average

shift:____________hrs.____________mins.

Examples of heavy work are lifting a heavy load (about 50 lbs.) from the floor to your waist or

shoulder; working on a loading dock; shoveling; standing while bricklaying or chipping castings;

walking up an 8-degree grade about 2 mph; climbing stairs with a heavy load (about 50 lbs.).

13. Will you be wearing protective clothing and/or equipment (other than the respirator) when

you're using your respirator: Yes/No

If "yes," describe this protective clothing and/or equipment:__________

_______________________________________________________________________

14. Will you be working under hot conditions (temperature exceeding 77 deg. F): Yes/No

15. Will you be working under humid conditions: Yes/No

16. Describe the work you'll be doing while you're using your respirator(s):

_______________________________________________________________________

_______________________________________________________________________

17. Describe any special or hazardous conditions you might encounter when you're using your

respirator(s) (for example, confined spaces, life-threatening gases):

_______________________________________________________________________

_______________________________________________________________________

18. Provide the following information, if you know it, for each toxic substance that you'll be

exposed to when you're using your respirator(s):

Name of the first toxic substance:_____________________________________

Estimated maximum exposure level per shift:____________________________

Duration of exposure per shift_________________________________________

Name of the second toxic substance:____________________________________

Estimated maximum exposure level per shift:____________________________

Duration of exposure per shift:________________________________________

Name of the third toxic substance:_____________________________________

Estimated maximum exposure level per shift:____________________________

Duration of exposure per shift:________________________________________

The name of any other toxic substances that you'll be exposed to while

using your respirator:

_______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________

19. Describe any special responsibilities you'll have while using your respirator(s) that may affect the safety and well-being of others (for example, rescue, security):

Appendix D (Mandatory) Information for Employees Using Respirators When Not Required

Under the Standard

Respirators are an effective method of protection against designated hazards when properly

selected and worn. Respirator use is encouraged, even when exposures are below the exposure limit, to provide an additional level of comfort and protection for workers. However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the worker. Sometimes, workers may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards. If your employer provides respirators for your voluntary use, of if you provide your own respirator, you need to take certain precautions to be sure that the respirator itself does not present a hazard.

You should do the following:

1. Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning

and care, and warnings regarding the respirators limitations.

2. Choose respirators certified for use to protect against the contaminant of concern. NIOSH, the National Institute for Occupational Safety and Health of the U.S. Department of Health and

Human Services, certifies respirators. A label or statement of certification should appear on the

respirator or respirator packaging. It will tell you what the respirator is designed for and how

much it will protect you.

3. Do not wear your respirator into atmospheres containing contaminants for which your

respirator is not designed to protect against. For example, a respirator designed to filter dust

particles will not protect you against gases, vapors, or very small solid particles of fumes or

smoke.

4. Keep track of your respirator so that you do not mistakenly use someone else's respirator.